This zip file contains the following:

| WeatherStations_V2025A_BTMPV_VW2025D_ OFFSHORE_LatLon | Identifies latitude and longitude associated with each weather station |
| --- | --- |
| MapWeatherDataToStation_V2025A_BTMPV_ VW2025D_OFFSHORE | Maps each weather data (8760 normalized profile for weather years 2000-2022) to a weather station |
| NormalizedWY2000-2022_SOLARV2025A_ BTMPV_# | Files containing solar weather data (8760 normalized profile for weather years 2000-2022). The files are broken up into many files that are small enough to be opened in Excel. Use Python or similar tools to re-append data. |
| NormalizedWY2000-2022_WIND_VW2025D_ OFFSHORE | File containing offshore wind weather data (8760 normalized profile for weather years 2000-2022) |

The Baseline Generator List and the Portfolio/Use-case-specific generator lists include columns that identify the weather station associated with a particular unit. The actual hourly production profile for that particular unit can be built up from the normalized weather data as follows:

- Identify the weather station for the unit (from one of the generator lists). For example, the unit ADOBEE_1_SOLAR has weather_station = SOLAR_1AXIS_CA_KERN_BAKERSFIELD_V2024.

- Identify the relevant technology attributes of the unit. ADOBEE_1_SOLAR has Unit Category = Solar_1Axis, meaning utility-scale single axis tracking. Utility-scale units (one axis, two axis, and fixed) are assumed to have inverter loading ratio = 1.3, meaning the panels are assumed to be oversized relative to the inverter. If the installed capacity (capmax) is 100 MW, the panels are 130 MW. For Unit Category BTMPV, the assumed inverter loading ratio = 1.13.

- Match the weather station to the normalized hourly weather data using the file MapWeatherDataToStation_V2025A_BTMPV_VW2025D_OFFSHORE. A weather station will be associated with 23 years of weather data (2000-2022).

- Normalized hourly weather data are integer values generally between 0 and 100. Divide all hourly values of step 3, for all weather years, by 100 to convert to values between 0 and 1.

- Multiply all hourly values in step 4 by capmax of the unit, e.g. 100 MW.

- Adjust for technology attribute if needed. If inverter loading ratio = 1.3, multiply all hourly values by 1.3 but cap values at capmax of the unit, e.g. 100 MW.

- The final hourly set should now be 23 weather years of production profiles corresponding to this particular unit.

- The steps for wind units are the same except skip steps 2 and 6.