BEFORE THE

PUBLIC UTILITIES COMMISSION

OF THE

STATE OF CALIFORNIA

Order Instituting Rulemaking into       )

Implementation of Pub. Util. Code #390  )    R. 99-11-022

)

Prepared Rebuttal Testimony of R. Thomas Beach

on Line Loss Issues

on behalf of the California Cogeneration Council

Jerry Bloom, Esq.

Joseph M. Karp, Esq.

White & Case, LLP

633 West Fifth Street

Los Angeles, California 90071

Telephone: 213-620-7700

Facsimile: 213-687-0758

e-mail: jkarp@whitecase.com

May 8, 2000

Subject Index

I.   SOUTHERN CALIFORNIA EDISON. . . . . . . . . . . . . . . . .3

A.   Edison's "New Market Entrant" SRAC Pricing Methodology is Inconsistent with

its Proposal to Use GMMs as the Line Loss Adjustment .3

B.   The Commission Should Ignore Edison's Calculation of Past Overpayments to

QFs Due to Allegedly Too-High Line Loss Factors. . . .4

II.  OFFICE OF RATEPAYER ADVOCATES . . . . . . . . . . . . . . .6

A.   Scaled GMMs Do Not Fully Reflect Avoided Line Losses, and Thus Are Not a

Reasonable or Adequate Measure of the Losses that a QF Avoids6

III. SAN DIEGO GAS AND ELECTRIC. . . . . . . . . . . . . . . . .8

A.   SDG&E's Line Loss Study Supports the CCC's Proposed Methodology8

IV.  CAITHNESS ENERGY. . . . . . . . . . . . . . . . . . . . . 11

A.   As An Alternative to the CCC's Line Loss Methodology, the Commission Could

Maintain the Present QF Line Loss Factors. . . . . . 11

BEFORE THE

PUBLIC UTILITIES COMMISSION

OF THE

STATE OF CALIFORNIA

Order Instituting Rulemaking into       )

Implementation of Pub. Util. Code #390  )    R. 99-11-022

)

Prepared Rebuttal Testimony of R. Thomas Beach

on Line Loss Issues

on behalf of the California Cogeneration Council

Q:   Please state for the record your name, position, and business address.

A:   My name is R. Thomas Beach. I am principal consultant with the firm Crossborder

Energy. My business address is 2560 Ninth Street, Suite 316, Berkeley, California 94710.

Q:   Please describe your experience and qualifications.

A:   My experience and qualifications are described in Attachment RTB-1 to the direct

testimony that I filed in this proceeding on February 14, 2000.

Q:   Have you testified previously in this case?

A:   Yes, I have.  In this case, I have filed direct and rebuttal testimony on the criteria for a

"properly functioning" market and on short-run avoided cost (SRAC) pricing issues under

Pub. Util. Code Section 390.  I served this direct and rebuttal testimony on February 14

and March 6, 2000, respectively.  This testimony was submitted on behalf of the California

Cogeneration Council (CCC) and the Watson Cogeneration Company (Watson).  On April

28, 2000, I served direct testimony on line loss issues on behalf of the CCC, as well as

supplemental testimony describing a comparison exhibit that I prepared on behalf of both

the CCC and Watson.

Q:   On whose behalf are you presenting this rebuttal testimony today?

A:   I am appearing on behalf of the California Cogeneration Council (CCC).

Q:   What is the scope of this rebuttal testimony?

A:   I am responding to the direct testimony of several parties on line loss issues, which was

served on April 28, 2000.  I comment on an inconsistency between Southern California

Edison Company's (Edison) proposed line loss adjustment for qualifying facilities (QFs)

and Edison's proposal for short-run avoided cost (SRAC) energy pricing.  I also rebut

Edison's attempt to quantify past overpayments to QFs due to allegedly inaccurate line

loss adjustments.  I also comment on an assertion in the testimony of the Office of

Ratepayer Advocates (ORA) that the use of the California Independent System Operator's

(ISO) Generation Meter Multipliers (GMMs) as the QF line loss adjustment is "adequate."

In addition, I critique the line loss study presented by San Diego Gas and Electric

Company (SDG&E).  Finally, I present the CCC's reaction to the line loss testimony of

Caithness Energy, L.L.C. (Caithness).

Q:   Three parties   Edison, SDG&E, and ORA   recommend the use of the ISO's

GMMs alone as the QF line loss adjustments.  Have you addressed the problems

associated with the use of GMMs for this purpose?

A:   Yes, I have.  My direct testimony dealt at length with the reasons why the GMMs alone

do not reflect the line losses that QF power avoids.  I will not repeat that testimony here,

except to emphasize once again the key shortcoming of using the GMMs alone: the

GMMs simply allocate a share of system average losses to the QF, and fail to reflect the

incremental impact of the QF on overall system losses, as PURPA requires.

I.   SOUTHERN CALIFORNIA EDISON

A.   Edison's "New Market Entrant" SRAC Pricing Methodology is Inconsistent

with its Proposal to Use GMMs as the Line Loss Adjustment.

Q:   Edison proposes to use the ISO's GMMs alone as the QF line loss adjustment.

Edison also has proposed what it calls the "new market entrant" approach to SRAC

pricing.   Are these proposals consistent with each other?

A:    No.  Edison's proposal to use GMMs for the QF line loss adjustment is inconsistent with

one of the details of its proposal to use the production costs of a new market entrant   a

new gas-fired combined-cycle plant   as the basis for short-run avoided cost (SRAC)

prices.

Q:   Please explain this inconsistency.

A:   Edison argues that the GMM must be applied to the QF's generation in order to reflect the

amount of power for which "the PX acknowledges receipt."  Edison April 28 Testimony,

at page 9, line 26.  In other words, the QF's generation must be adjusted by the GMM in

order to reflect the amount of power that the PX deems to have been delivered through

the transmission system to the load center.  Ibid, page 9, lines 3-4.  Under Edison's

approach, the SRAC energy price that the QF receives thus must be a price calculated at

the load center, and the total SRAC energy payment that the QF receives should be the

SRAC price (at the load center), times the QF's generation, times the QF's GMM.

For the SRAC price, Edison has proposed to use the production costs of a new

market entrant.  As set forth in Exhibit 50 (page 45, lines 15-17 and Table 1 on page 46),

Edison would calculate this cost as the new entrant's heat rate times its burnertip gas cost

times a GMM of 0.98, plus variable O&M and energy-related capital costs.  However,

this is not the way a new entrant would treat its GMM in calculating its production costs.

The new plant must face the fact that the ISO will apply a GMM to its production in

determining how much of the new entrant's power will reach the load center.  Thus, if the

new entrant wants to receive its full production cost for the power it generates, it must bid

into the PX a price equal to its per unit production costs divided by its GMM.  When this

bid is accepted, the new entrant will receive a total payment no lower than its bid price

(per unit production costs divided by its GMM) times its generation times its GMM, i.e.

its full production costs.  Thus, the correct SRAC price at the load center under the new

entrant approach is not the new entrant's production costs times its GMM (as Edison

proposes), but the new entrant's production costs divided by its GMM.  Based on the

exemplary GMM of 0.98 that Edison uses, Edison's new market entrant approach thus

understates the avoided energy cost of the new entrant's power at the load center by one

divided by (0.98)2, or 4.1%.

Thus, if Edison desires to use GMMs as the QF line loss adjustment, then Edison

must use the GMMs consistently in its SRAC pricing proposal.  The new market entrant's

production costs should be divided by its GMM, not multiplied by the GMM.  In addition,

the use of GMMs will introduce yet another source of controversy into the implementation

of Edison's new market entrant approach: determining which new entrant, and thus which

GMM, to use as the basis for SRAC pricing.  QFs undoubtedly will seek to use a very

remote new entrant with a low GMM (which would benefit QFs by raising the new market

entrant's costs to deliver power to the PX), while the IOUs will argue that the

Commission should use a new load center project with a high GMM (and thus lower

production costs for power delivered to the PX).

B.   The Commission Should Ignore Edison's Calculation of Past Overpayments

to QFs Due to Allegedly Too-High Line Loss Factors.

Q:   Edison's April 28 testimony includes testimony from Mr. Davis that calculates that

QFs have been over-paid $1.24 billion since 1982 due to line loss factors that Edison

asserts have been too high.  How much weight should the Commission give to this

calculation?

A:   None at all.  Once again in this case, Edison has tried to re-write history in a completely

unrealistic fashion.  Edison's calculation is based on comparing QF payments from 1982 -

98 using the transmission loss factors in effect since 1982 (1.0 in 1982 - 83 and 1.023

thereafter) to the QF payments that would have been made over those seventeen years

using the average GMM in 1999 times the distribution loss factor in 1999 (0.974).  In

effect, this assumes that, somehow, the electric industry could have been restructured in

1982 such that GMMs would have been available beginning at that time.  It also assumes

that the GMMs that would have applied in 1982 are similar to those that the ISO

calculated in 1999, even though at least two other parties (SDG&E and Caithness) have

testified that power flows on the IOUs' transmission systems have changed greatly since

1982.  Finally, Edison's calculation assumes that the GMMs are the proper measure of

avoided line losses   a proposition that I strongly dispute, as set forth in my direct

testimony.

In my view, it is just as likely that QF line loss factors have been set too low over

the past 17 years.   Caithness correctly points out that, from a long-run perspective,

without QFs the California IOUs probably would have built additional central station

power plants, either coal plants located far outside of California or nuclear plants at

remote locations within the state.  In establishing the line loss adjustment for QFs paid

based on long-run avoided costs (for example, for Interim Standard Offer No. 4 QFs), the

Commission probably should have reflected the line loss savings that resulted from in-state

QF purchases, compared to the avoided alternative of generation at the out-of-state coal

or remote nuclear plants that were never built.  These line loss factors could easily have

been much higher than the factors that actually have been in place since 1984.

Again, the bottom line is that it is a waste of the Commission's time to engage in

such historical disputes, which contribute nothing to moving forward on the important

issues at hand.

II.  OFFICE OF RATEPAYER ADVOCATES

A.   Scaled GMMs Do Not Fully Reflect Avoided Line Losses, and Thus Are Not

a Reasonable or Adequate Measure of the Losses that a QF Avoids.

Q:   ORA asserts that the GMMs, which reflect scaled marginal losses, are "a reasonable

measure of costs or savings resulting from variations in line losses."  ORA Testimony,

at page 6, lines 24-25.  Do you agree?

A:   No.  ORA advances two dubious arguments in support of its point.  First, ORA asserts

(without supplying any data) that QFs increase losses in Edison's service territory.  In

other words, the aggregate GMM for all Edison QFs is lower than the system average

GMM.  ORA implies that the use of scaled GMMs benefits QFs because the scaling

reduces the additional line loss costs attributable to Edison QFs.  However, my review of

the aggregate GMMs for PG&E and SDG&E QFs shows that those aggregate GMMs are

higher than the system average GMM.  Thus, QFs in PG&E's and SDG&E's service

territories clearly reduce overall line losses.  Thus, the use of GMMs would reduce the line

loss benefits that should be paid to PG&E and SDG&E QFs.

ORA's second point is that the understatement of losses due to the scaling process

is reasonable "because the measure of costs is adequate."  ORA appears to be saying that

the use of scaled GMMs is reasonable because losses are just a small part of overall QF

energy payments.  Just because the loss adjustment for QF energy payments is typically

less than 5% of the total payment does not justify the use of a loss adjustment that

typically is wrong by a factor of about two (the typical scale factor for GMMs).  SRAC

prices should send accurate price signals to QFs, and by law must reflect the full avoided

costs or savings resulting from variations in line losses due to the presence of a QF.  As

set forth in my opening testimony, it is relatively easy to calculate a QF line loss

adjustment, using GMMs, that corrects for the scaling process.  The Commission should

not adopt the use of GMMs simply because they are "adequate" or "good enough," when

a better, more accurate method is readily available.

Q:   ORA attempts to suggest that SDG&E's line loss study justifies the use of GMMs

for all three IOUs.  ORA Testimony, at page 7, lines 23 - 27.

A:   I comment in Section IV of this rebuttal testimony on SDG&E's line loss study.  In my

opinion, SDG&E's study does not support the use of GMMs alone.  My testimony below

shows that, based on the SDG&E study, the line loss factors for SDG&E's QFs, if

correctly calculated on an incremental basis as PURPA requires, are significantly higher

than the GMMs for those QFs.  The CCC's recommended line loss methodology actually

produces line loss factors that come the closest to those in SDG&E's study.  I note that

SDG&E's April 28, 2000 line loss testimony no longer uses its study as a justification for

recommending the use of GMMs.

In addition, ORA is clearly reaching too far in its assertion that the SDG&E study

shows that comparable studies for PG&E and Edison also would produce results that

justify the use of the GMMs.  ORA Testimony, at page 8, lines 3-4.  There are major

differences between the SDG&E electric system and those of Edison and PG&E, in terms

of (1) the number of QF projects, (2) the location of QF projects, (3) the geography of the

IOU's transmission system, and (4) the power flows on the IOU system.  It is simply too

speculative to say what the results of similar studies for PG&E and Edison would be.

III. SAN DIEGO GAS AND ELECTRIC

A.   SDG&E's Line Loss Study Supports the CCC's Proposed Methodology.

Q:   SDG&E's testimony includes a line loss study that is specific to its service territory.

SDG&E asserts that the study supports the use of GMMs for QFs in the San Diego

area.  Do you have any comments on this study?

A:   As shown in Table 8 of Mr. Abed's testimony, SDG&E's study produces energy loss

factors in the range of 1.003 to 1.007 for four of SDG&E's larger QFs.  I have serious

concerns about one element of SDG&E's methodology   the so-called "conversion

factor" that Mr. Abed applies to convert "capacity" loss factors to "energy" loss factors.

As I understand SDG&E's approach, the utility used a power flow model to

examine the incremental losses which result from removing each of four QFs from its

system.  SDG&E did this for a range of system demand conditions (peak load, medium

load, and light load), as shown in Mr. Abed's Tables 4 - 6.  SDG&E then calculated a

"capacity loss factor" (Table 7) as one plus the ratio of the incremental losses in each case

to the size of the QF.  Finally, to determine an "energy loss factor" (Table 8) Mr. Abed

applied a so-called "conversion factor" of 0.39558 to the ratio of the incremental losses to

the capacity of the QF.  The use of this "conversion factor" results in energy loss factors in

Table 8 that are much smaller than the capacity loss factors in Table 7.

I question Mr. Abed's use of this "conversion factor."  He states that this

"conversion factor" should be used to calculate energy loss factors in order to incorporate

"the effect of the system load variance over the period of time that is being considered for

the calculation of the loss factor."  Testimony of A.M. Abed, at page 7, lines 1 - 2.

However, Mr. Abed's method already captures the impact of system load variations, by

examining three separate cases that span a range of load conditions (peak, medium, and

low), and then taking a weighted average of the loss factors over the three cases.   Mr.

Abed himself states that these three cases "encompass the full range of load variation

expected."  Ibid., page 10, lines 6-7.  Having already examined a range of load conditions,

there is no need to apply an additional "conversion factor" that also is intended to capture

the impact of variations in system loads.

In discovery, the CCC requested a copy of the reference text that Mr. Abed cites

as the source for his "conversion factor."   The relevant pages that Mr. Abed cites are

included as Attachment RTB-1 to this testimony.  Mr. Abed's "conversion factor" is

actually the "loss factor" discussed in Section 11 of this text.  The text nowhere discusses

the use of this "loss factor" to convert "capacity" losses to "energy" losses.  This "loss

factor" is defined in the opening sentence of Section 11 of the text as "the ratio of the

average power loss to the peak load power loss, during a specified period of time."  The

text then describes this factor as indicating "the degree to which the load loss within the

apparatus during peak load is maintained throughput the period in which the loss is being

considered."  In other words, this factor allows you to estimate the average power loss in

a line over a certain period, if all that you know is the peak power loss and the load factor

of the circuit over the same period.  The use of this "loss factor" might have been

appropriate if Mr. Abed had only calculated the incremental losses from these QFs during

SDG&E's system peak   in other words, if Mr. Abed had run his load flow analyses just

for the Peak Period case.  Then he would have needed an adjustment like this "loss factor"

to estimate average losses over the full range of annual demand conditions on SDG&E's

system.  But Mr. Abed removed the need to use such a "loss factor" adjustment when he

examined the two other cases (Medium and Light Loads), which provided direct

information on the incremental losses over the full range of annual demand conditions on

SDG&E's system.

Mr. Abed's use of a "conversion" factor to go from "capacity" to "energy" loss

factors thus represents an inappropriate, duplicative adjustment for variations in load

conditions on the SDG&E system.  Mr. Abed should have ended his study after

calculating what he calls the "capacity loss factors" in his Table 7.  These loss factors

represent the incremental variations in line losses on the SDG&E system due to the

presence of these four QFs   exactly what the FERC rules specify as the QF line loss

adjustment.  Based on Mr. Abed's work, the loss factors in his Table 7 represent the

correct values for the line loss adjustments for SDG&E's QFs.  In my view, it makes

complete sense that the numbers in Mr. Abed's Table 7 are higher than the GMMs for

these QFs, which are all just slightly above or below 1.0.  The loss factors in Table 7 are

based on the incremental, unscaled losses due to the presence of these QFs; they should

be   and are   significantly higher than the scaled GMMs for these QFs.  The higher

numbers in Table 7 are similar to the aggregate line loss factor of about 1.015 that the

CCC's recommended methodology produces for SDG&E QFs, as noted in my direct

testimony.  Thus, in my view, Mr. Abed's study   corrected to remove his use of the

duplicative "conversion factor"   supports the CCC's recommended approach to QF line

loss adjustments.

IV.  CAITHNESS ENERGY

A.   As An Alternative to the CCC's Line Loss Methodology, the Commission

Could Maintain the Present QF Line Loss Factors.

Q:   Caithness recommends the continued use of the present QF line loss factors "unless

and until a clearly superior methodology emerges."  What is your reaction to this

proposal?

A:   Caithness's testimony accurately depicts many of the problems and uncertainties

associated with the use of GMMs alone as the QF line loss adjustment.  The CCC's

methodology has attempted to remedy several of these problems, including the failure to

comply with PURPA and the distortions caused by the scaling of the GMMs, while still

making use of the marginal loss information that is contained in the GMM data.  If,

however, the Commission concludes that any GMM-based line loss adjustment is too

problematic to adopt at this time, then the CCC would support Caithness'

recommendation to maintain the present QF line loss factors.

Q:   Does this conclude your rebuttal testimony?

A:   Yes, it does.