2007 Filing Guide for System and Local Resource Adequacy (RA) Compliance Filings

1.	The Filing Process and the Local RAR	2

2.	Confidentiality and RA Filings	3

3.	Qualifying Capacity	4

4.	Maximum Cumulative Capacity and Resource Categories	6

5.	Demand Response (DR) Resources	6

6.	Outages	7

7.	Allocation of Reliability Must Run (RMR) Units for Local and System RAR	8

8.	Import Capacity Allocation Process for 2007	10

9.	DWR Contracts	13

10.	Liquidated Damages (LD) Contracts	13

11.	Resource Adequacy (RA) Portfolio Resources	14

12.	Certification of LSE Resource Adequacy Compliance Filing	14

13.	RA Filings Instructions	15

14.	Correction of Errors: Minor or Substantial	16

Appendix A – Primer for Filing an Advice Letter

Appendix B – Advice Letter Summary Form

Appendix C - SC/LSE and CAISO Required Tasks

Appendix D – Frequently Asked Questions

- The Filing Process and the Local RAR

Decision (D.) 05-10-042 established a year-ahead and monthly System Resource Adequacy Requirement (RAR) for load serving entities (LSEs) under the jurisdiction of the California Public Utilities Commission (CPUC).  D. 06-06-064 expanded the RA program to include a year-ahead Local RAR and a Preliminary Local RAR for compliance year 2007.

- The Preliminary Local RAR compliance filing requires LSEs to demonstrate if they have procured any unit that is designated as eligible for reliability must run (RMR), or is on the CAISO’s Local Area Resource Data List.  The preliminary Local RAR covers the full 12-month period of 2007.  LSEs should not limit the showing to units sufficient to meet load or located within areas in which they have been allocated Local RAR. Due September 22, 2006.

- LSEs are required to make a 2007 year-ahead System and Local RAR compliance filing that demonstrates compliance with the 90% of system RAR obligation for the five summer months of May through September 2007, as well as 100% of the Local RAR for all 12 months of 2007. LSEs must include in the local showing any unit that is on the California Independent System Operator Local Area Resource Data List or the list of RMR eligible resources.  The Local RAR covers the full 12-month period of 2007.  Due October 31, 2006.

- LSEs are required to continue making monthly forecasts and monthly system RAR showings that track load migration and demonstrate compliance with the 100% system RAR.  There is no monthly Local RAR or required Local RAR monthly filing.  Due monthly with no set end date.  For more information on the2006 Monthly RAR Filings please see the “Resource Adequacy Guide for Monthly Filings” issued March 29, 2006.  More information is forthcoming explaining 2007 Monthly RAR Filings.

Templates

The RAR Filing Guide herein and the accompanying RAR reporting templates (Standard and Local Only templates) provide the means for LSEs to demonstrate compliance with the year-ahead System RAR, as well as the Local RAR. LSEs should use the following templates to demonstrate compliance:

- For 2007 Preliminary Local RAR filing – LSEs should use the Local Template to demonstrate Preliminary Local RAR for all 12 months.  LSEs must demonstrate whether they have any unit under contract that is either identified on the Local Area Resource Data List under contract for 2007 or any unit with a 2006 RMR contract. LSEs may also show any other unit under contract by the date of the filing, even if it is not in a local area.  The CAISO will consider all units under contract (even those outside local areas) before making final determinations for RMR contracts for 2007.

- For 2007 System RAR – LSEs should use the standard template for the period of May through September (This item is a repeat of the 2006 year-ahead System RAR process.)

- For 2007 Local RAR – LSEs should use the Local RAR Template to demonstrate compliance with the Local RAR for all 12 months.

Notification of RAR by Energy Division

Each LSE will be notified by the CEC/CPUC Energy Division of their Local RAR and System RAR.  This notification process will consist of three parts.

- For System RAR – LSEs were notified of their adjusted forecast System load on July 12, 2006.  Each LSE’s System RAR is 115% of this expected peak load by month.  LSEs received their provisional Condition 2 allocation notifications to aid them in preparing their System RAR Filing on July 26, 2006, and their final Condition 2 RMR allocation on or about October 6, 2006.  On a separate timeline, each LSE will receive notification of their import allocation for use in their System RAR filing.  See Section 7 for more details regarding Import Allocation.

- For Local RAR -- LSEs were notified of their final Local RAR by the CEC via certified letter on July 26th  based on the methodology presented at the July 21, 2006 Energy Division workshop.  LSEs also received their Provisional RMR Condition 2 Allocation at the same time.  LSEs will receive Condition 1 RMR credits by October 6, 2006.

- For Monthly System RAR – LSEs were issued their System RAR for all months of 2007 on July 12, 2006.  LSEs are expected to comply with the Monthly Load Forecast Adjustment process throughout 2007 as done during 2006.  LSEs received their provisional Condition 2 allocation notifications on July 26, 2006, and will receive a confirmation of the Condition 2 RMR allocation on or about October 6, 2006.  Energy Division will notify LSEs of any change to RMR allocations as they occur throughout compliance year 2007 for use in subsequent Monthly RA Filings.

- Confidentiality and RA Filings

D.06-06-066 revised the Commission's confidentiality procedures and placed the burden of justifying protection of confidential information on the party asking for the protection, not the party that wants the information released.  Pursuant to this decision, the Revised Protective Order issued in R.05-12-013 on February 10th will undergo changes.  Key among the changes is a revised designation of which material can be kept confidential, and the method for protecting such information.  Certain classes of information are described in the matrices attached to D.06-06-066; the matrices break the information down into specific types that are to be public and specific types that are to be confidential.  There is also a description of the purpose and requirements for a motion to protect information included on page 24 of D.06-06-066.

LSEs must file a motion to retain confidential treatment for any information contained in RA filings, including Monthly RA filings, Preliminary Local RAR showings, System RA filings, and Local RAR filings.  In response to a motion to request confidentiality, the Administrative Law Judge may find some parts of the RA Filings to still be confidential.  LSEs should match the type of information contained in the RA Filings against the matrices in Appendix 1 and 2 in D.06-06-066 and prepare a motion accordingly.  LSE may file one motion to cover the four types of RA filings.  To the extent that the types of information required in the RA Filings remain constant, no further motion will be required to protect the same information.  To protect the confidentiality of future RA Filings, LSEs will be required to cite any rulings on the original motion in the Advice Letters that accompany them however.

Protection of any information to be designated as confidential shall be requested and justified with a motion that proves the following:

- That the material it is submitting constitutes a particular type of data listed in the Matrix,

- Which category or categories in the Matrix to which the data correspond,

- That it is complying with the limitations on confidentiality specified in the Matrix for that type of data,

- That the information is not already public, and

- That the data cannot be aggregated, redacted, summarized, masked or otherwise protected in a way that allows partial disclosure.

D.06-06-066 is effective as of the mailing date of July 4, 2006 but in order to give LSEs time to understand the decision and prepare the required motion, LSEs are not required to file motions for protection of RA Filing information until the September 22, 2006 Preliminary Local RA Filing date.  Information contained in RA Filings made after September 21, 2006 will be considered public unless a motion is filed requesting confidentiality and the motion is approved.

The Administrative Law Judge will review each motion and issue a ruling addressing requests for protection of information in the RA Filings.  LSEs are all encouraged to file one joint motion as all LSEs file information on identical templates, and a joint motion would minimize workload.  LSEs will be notified when additional information is to be included in the RA Filings, and when any additional motions will be required.

- Qualifying Capacity

D. 05-10-042 requires all LSEs to fulfill their allocated system RAR through purchase of Qualifying Capacity (QC).  The Net Qualifying Capacity for Compliance Year 2007 (“QC List”) list is available on the CAISO website at: http://www.caiso.com/1796/179688b22c970.html.  Please refer to this list when identifying a resource with Scheduling Resource ID and its associated QC.

The QC list that was published on July 14, 2006 covers compliance year 2007.  The QC values on this list will not be changed once it is issued, except for data maintenance such as correction of errors and addition of new resources that come online during the course of 2007.  Any revisions made by the CAISO after July 14, 2006 can raise a given unit’s QC, but revisions cannot lower it.

QC and Local RAR

D.06-06-064 requires all LSEs to procure physical resources to meet the Local RAR. These units are to be located in the nine local areas identified in the CAISO Locational Capacity Technical Analysis.  For purposes of compliance, the nine local areas have been aggregated into four areas (LA Basin, San Diego, Other PG&E Local Areas, and the Greater Bay Area).  The Other PG&E Local Areas include the Local Areas of Fresno, Humboldt, Kern, North Coast/North Bay, Sierra, and Stockton.  The LSE is responsible for verification of the Local Area Designation of the unit, as well as the QC value and the Scheduling Resource ID.  QC that is to be entered for compliance with the Local RAR is either 1) the QC listed, for resources that have only one annual QC value or 2) the August QC value for resources that have monthly QC values which reflects performance of the unit at IOU Service Area Peak.  To locate a unit within a Local Area, please refer to the 2007 Local Area Resource Data list available on the CAISO website at: http://www.caiso.com/1796/179688b22c970.html.

Every resource has a resource name (“RES Name”) and associated resource identification (“Scheduling Resource ID”).  The Scheduling Resource ID in the Net Qualifying Capacity for Compliance Year 2007 list can be cross-referenced against the Scheduling Resource ID (“SCHED RES ID”) column found in the 2007 Local Area Resource Data list to identify within which if any Local Area the unit is located.  If a resource from the QC List cannot be found in the 2007 Local Area Resource Data, then that resource is not located within a local area and can only count toward the System RAR.

To report a contract with a unit on the Local Area Resource Data list, LSEs are required to select from a drop down list of local areas on the template.  The template gives the choices of LA Basin, San Diego Area, Other PG&E Local Areas, or Greater Bay Area.  All units within any of the Other PG&E Local Areas are to be entered as Other PG&E Local Areas, not as Humboldt, Fresno, Kern, etc.  Local RARs above 1 MW are rounded to the nearest MW while obligations less than 1 MW receive an exemption from Local RAR procurement.

QC and Outages

While outages do not change a units QC value during an RA compliance year, LSEs are expected to verify with the Scheduling Coordinator the availability of any units the LSE wishes to include in their Monthly System RA Filings, and compute the appropriate amount of capacity that is affected pursuant to the formulas in Section 6 below.  The Local RAR Filings will not be affected by either scheduled or forced outages.

- Maximum Cumulative Capacity and Resource Categories

The 2007 Local and System RAR Filings will use the resource eligibility criteria developed for the 2006 year-ahead System RA filing through the December 9, 2005 workshop.  The Maximum Cumulative Capacity (MCC) percentages established for the 2006 filings will still be used for 2007,  but for purposes of the 2007 year ahead filings, the MCC restrictions will be based on the 115% RAR, not the 90% year ahead RAR as before.  This change will provide more consistency with the Monthly RA filing process..

| Summary of Resource Categories |  |
| --- | --- |
| Category | Consensus Agreement Resources may be categorized into one of the four categories shown below, according to their planned availability as expressed in hours available to run or operate per month (hours/month): |
| 1 | “Greater than or equal to” the ULR [Use Limited Resource] monthly hours as shown in the Phase 1 Workshop Report, Table “Number Hours ISO Load Greater than 90% of the Monthly Peak,” p.24-25, last line of table, titled “RA Obligation,” http://www.cpuc.ca.gov/word_pdf/REPORT/37456.pdf These ULR hours for May through September are, respectively: 30, 40, 40, 60, and 40, which total 210 hour and have been referred to as “the 210 hours.” |
| 2 | “Greater than or equal to” 160 hours per month. |
| 3 | “Greater than or equal to” 384 hours per month. |
| 4 | All Hours (planned availability is unrestricted) |

- Demand Response (DR) Resources

Dispatchable Demand Response (DR) resources shall be ‘taken off the top’ of the RAR and entered into a separate DR worksheet that will not have a maximum cumulative contribution (MCC) percentage.  The amount of capacity in megawatts (MW) entered into the DR worksheets will be multiplied by 115% percent to reflect their status as dispatchable load based resources and will be subtracted from the overall 115% System RAR.  For example, if LSE-ABC has a 1,000 MW RAR in June 2007, and LSE-ABC shows 100 MW of DR resources in the DR bucket, LSE-ABC’s remaining obligation is 900 MW, which must be met with an acceptable combination of resources.

For DR programs that are operated by the IOUs and where cost is allocated across all transmission customers, the RA counting credit for these programs will also be allocated across all LSEs in a transmission service area based on share of program cost that customers of the LSE have paid.  CEC Allocations of DR resources are all allocations for programs that exceed 2 hours a day maximum, and thus should be recorded in the System RA template on the DR-a_2hr-Plus sheet.  LSEs that receive allocations for programs which they do not administer need only include the total summary information from the CEC Allocation, and the total MW amount of QC.  LSEs are required to provide all program details for DR programs which they themselves operate.

D.06-06-064 determined that dispatchable DR credit should be allocated to each Local Area and counted for the Local RAR, but that it may not be possible for the 2007 compliance year.  CEC and Energy Division staffs are pursuing the collection of data to accomplish this allocation, but do not expect to implement it for the 2007 compliance year.  If the allocation is completed, LSEs will be so informed and provided with the appropriate allocations.

Non-dispatchable demand response resources are considered a reduction in demand and were included in the Load Forecasts.  Therefore, they are not to be listed as resources in the System RA Filing.

- Outages

D. 06-07-031 adopted the following system for the counting of scheduled and forced outages for purposes of the System Monthly RA Filing.  Please note that the Local RA Filing will not be affected by scheduled or forced outages and the following section and table apply only to the System RA Filings, not the Local RA Filings.

Scheduled Outages:

For RA counting purposes, a Scheduled Outage is any outage that is designated "Approved Planned" in the SLIC system.   In determining the impact of outages, LSEs should use the scheduled outage criteria in the table below.  LSEs are advised to verify a unit’s outage status with the Scheduling Coordinator for that unit prior to including it in the RA filing.  The CPUC and the CAISO will compare the amount of claimed capacity in the System year ahead and Monthly RA filings with the CAISO’s outage information to verify compliance.  An LSE’s filings may be disapproved if there is a variance between unit availability and the capacity listed in the System year ahead and Monthly RA filings.

All outage schedules and changes to outage schedules for a RA resource are subject to CAISO approval.  If the CAISO approves a change to the outage schedule, the outage schedule change will not change the RA counting of the resource.  If the CAISO denies an outage request, the RA resource is expected to remain available.

If a unit has a scheduled outage pursuant to the direction above that curtails only part of the unit’s capacity, a LSE may count for RA the capacity that is not curtailed.

Outages and the SLIC system

The magnitude in MW of curtailment and the length of an outage are both taken directly from the SLIC system.  Any unit that has an outage designated as “Approved Planned” in the SLIC system for a particular month should use the scheduled outage criteria in the table below to determine the impact of scheduled outages on that unit’s ability to count as Resource Adequacy Capacity.  Any LSE listing a resource in an RA Filing must confirm that the amount of capacity to be listed is not subject to curtailment prior to the filing date.

The amount of capacity that can count towards an LSE’s System RAR will be the total QC for the applicable filing month minus the computed curtailment amount.  The Energy Division will verify all Resource Adequacy Capacity against scheduled outage information in SLIC and will only allow capacity that is not under a scheduled outage to count towards all LSEs’ System RAR, subject to the scheduled outage counting rules in the table below.  Any approved changes to outages occurring after the filing due date will not affect an LSE’s compliance.

Counting Resources with Scheduled Outages

| Time Period | Description of How Resource Would Count at Time of the Showing |
| --- | --- |
| Summer May through September | Any month where days of scheduled outages exceed 25% of days in the month, the resource does not count for RAR. If scheduled outages are less than or equal to 25% the resource does count for RAR. |
| Non-Summer Months October through April | For scheduled outages less than 1 week, the resource counts for RAR. For scheduled outages 1 week to 2 weeks, the amount counted for RAR is prorated using the formula: [1 - (days of scheduled outage/days in month) - 0.25] * MW = RAR The formula will allow resources to count between 50% and 25%. For scheduled outages over 2 weeks, the resource does not count for RAR. |

Forced Outages:

Forced outage of a RA resource occurring during a month does not change the RA compliance established for that LSE for that month.  If the forced outage continues into a succeeding month, or months, the resource may still be counted towards the LSE's RA compliance.  The SLIC system verifies forced outages; units that, according to the SLIC system, have an outage designated as “forced” will not be derated for RA purposes and can sell capacity equal to the unit’s QC in the QC list.

# Allocation of Reliability Must Run (RMR) Units for Local and System RAR

D.06-06-064 requires that all LSEs file Preliminary Local RAR compliance showings on September 22, 2006.  This Preliminary Local RAR demonstration must, at a minimum, accurately show whether the LSE has, by September 22, 2006, entered into a contract with any unit that is among the list of units proposed for 2007 RMR Contracts as well as any unit that is listed on the 2007 Local Area Resource Data list.  The preliminary Local RAR demonstrations will not be used to determine compliance with local procurement obligations but they are required to be accurate.  In addition, LSEs are strongly encouraged to include any other units The LSEs are required to include other units in their Preliminary Local RAR demonstrations --including units not in designated local areas -- because complete data on LSE procurement may help to reduce RMR procurement and the associated costs that are allocated to LSEs.  An LSE that has no 2007 proposed units or units on the Local Area Resource Data List under an RAR contract prior to September 22, 2006 is required to so state in a simple filing; however, even the identification of non-local units may reduce RMR procurement, and therefore all LSEs are encouraged to be as complete as possible in Preliminary Local RAR Filings.

To the extent that an LSE’s Preliminary Local RAR showing contains any RMR units, the showing needs to indicate whether the LSE contracted for the RMR unit’s full RA capacity outright or contracted for only the Local RAR counting benefits via a “wraparound” contract to the RMR contract.  To qualify as providing Local RA, a wraparound contract must fully displace the RMR contract fixed costs such that no other transmission customers are paying for that part of the RMR contract obligation.

In requiring a Preliminary Local RAR demonstration on September 22, 2006, the CAISO will have a week to review the filings prior to issuing RMR contract renewal notices.  If a unit is bought under a full RA contract, there should be no CAISO RMR contract renewal.  If a unit is bought by wraparound contract, then the CAISO may still execute an RMR contract renewal but the Local RAR capacity credit of the RMR contract would not be distributed to all LSEs as Local RAR credit; instead the Local RAR counting benefit would accrue only to the LSE that is paying for the wraparound contract.  If an LSE has a wraparound contract that does not fully displace fixed cost recovery, the Local RAR credit would be allocated as if it were an RMR unit without any wraparound contract.  However, in that case the LSE that pays for the wraparound contract may still receive System RAR credit for the unit.

After finalizing the RMR contracts for 2007 by October 1, 2006, the CAISO will know which RMR units are available and the total MW of RMR capacity.  The CAISO, in coordination with the Energy Division, will then separate the total RMR contract amount (in MWs) by Commission-jurisdictional and non- jurisdictional, RMR-paying entities, and also by the four Local Areas.  Working with the Energy Division, and using the preliminary Local RAR filings, the CAISO will determine the amount of Commission-jurisdictional RMR that can be allocated as “Local RAR” credit to all the RMR-paying LSEs.  Each LSE will be notified by the Energy Division of their System RMR Allocation based on the expectation that any Condition 2 RMR unit that has a contract renewed by October 1, 2006 will likely continue to elect Condition 2 for 2007.  If there is any change to the Condition of a unit throughout the year, LSEs will be notified of an updated RMR Allocation for use in their Monthly RA Filings.

Both Condition 1 and Condition 2 RMR contracts will be allocated for purposes of Local RAR, while only Condition 2 units can be allocated to LSEs for purposes of System RAR.  D.06-06-064 specifically rejected the sale of separate System and Local RA counting credits; however RMR Condition 1 contracts that count towards Local RAR will not count towards System RAR.  Therefore, a unit that is under an RMR Condition 1 contract may separately sell its System RAR credit.

The following schedule summarizes the procedure:

Schedule for Coordinating RAR and RMR Processes for 2007

| June 29, 2006 | Commission adopts Local RAR Decision. |
| --- | --- |
| July 21, 2006 | Energy Division and CEC host staff workshop to discuss final CPUC-share of 2007 Local Capacity Requirement, divided by local areas and aggregated in accordance with decision. |
| July 25, 2006 | Energy Division notifies LSEs of their Local RAR for 2007 and their Provisional RMR Condition 2 system RA credit. |
| August 2006 | CAISO’s RMR study process complete. |
| September 6-7, 2006 | CAISO Board meets to consider RMR contract designations. |
| September 22, 2006 | LSEs file Preliminary Local RAR showings. |
| September 29, 2006 | CAISO finalizes (by October 1, 2006) its RMR contract renewal process by sending renewal notices. |
| October 6, 2006 | Energy Division notifies LSEs of their 2007 RMR credit for Local RAR. |
| October 31, 2006 | LSEs make year ahead System and Local RAR showings for 2007. |
| November 8, 2006 | CAISO notifies all LSEs of any collective deficiencies to allow for additional LSE procurement. |

| December 1, 2006 | Last date for LSE to file amended Local RAR or System “year ahead” RAR showing to reduce CAISO backstop for collective deficiency. |
| --- | --- |
| After December 1, 2006 | CAISO engages in any backstop required to cure collective local deficiencies and notifies the Energy Division so that it can notify LSEs of their System RAR credit to their monthly showings. |

- Import Capacity Allocation Process for 2007

Note:  Please refer to the CAISO Tariff, Section 40.5.2.2 for the express language on this topic and Appendix C of this Guide for a quick reference guide as to the timelines and tasks that are codified in this section of the CAISO Tariff.

The CAISO’s Tariff, section 40.5.2.2- Deliverability of Imports, outlines a seven (7)-step process necessary to allocate import capacity to all load serving entities.

In summary, import capacity will be assigned to entities that serve load in the CAISO Control Area in 2007 per the following steps:

- For 2007, the CAISO will establish for each branch group the total import capacity values for the CAISO Control Area and publish these values on its website.  This task has been completed and the information can be found on the CAISO website at: http://www.caiso.com/182c/182cb0f646060.pdf

- For each branch group, the total capacity established in Step 1 will be reduced by subtracting the import capacity associated with (i) Existing Transmission Contracts and (ii) Encumbrances and Transmission Ownership Rights.  Existing Transmission Contracts and Encumbrances and Transmission Ownership Rights will be reserved for holders of these commitments as part of the CAISO deliverability study and will not be subject to allocation under this Section of the CAISO Tariff.  The CAISO will publish on its website the quantity of remaining import capacity by branch group.  This task has been completed and the information can be found on the CAISO website at: http://www.caiso.com/182c/182cb0f646060.pdf

- Based on import capacity remaining on each branch group as determined in Step 2, Load Serving Entities that serve load within the CAISO Control Area requested an allocation of import capacity based on outside CAISO Control Area resource commitments entered into before March 10, 2006 and with a term lasting through the year 2007.  LSEs selected particular branch groups based on the primary branch group that energy or capacity from each particular import resource commitment has been historically scheduled or, for a resource commitment without a scheduling history, the primary branch group that the energy or capacity from the particular resource commitment is anticipated to be scheduled.  This task has been completed.

To the extent a particular branch group was over requested, such that the MWs represented in all requested resource commitments utilizing the branch group exceed the branch group’s remaining import capacity, the requested resource commitment MW quantities would be allocated capacity based on the “Import Capacity Load Share” ratio of each Load Serving Entity submitting such resource commitments.   However, to the extent this initial allocation had not fully assigned the total import capacity of a particular branch group to the requested resource commitments, the remaining capacity would be allocated until fully exhausted based on the Import Capacity Load Share ratio of each Load Serving Entity whose quantity of submitted resource commitment had not been fully satisfied.

- Import Capacity Load Share is each Load Serving Entity’s proportionate share of the forecasted 2007 coincident peak load for the CAISO Control Area relative to the total coincident peak load of all Load Serving Entities that have not had their request for import capacity for a resource commitment on a particular branch group fully satisfied.  The proportionate share of the forecasted 2007 peak load for the CAISO Control Area for each Load Serving Entity is the “Coincident Load Share” as determined by the California Energy Commission.  This task has been completed.

- The CAISO will notify the Scheduling Coordinator for each Load Serving Entity of the Load Serving Entity’s Coincident Load Share as well as the amount of import capacity on which branch group has been allocated to the Load Serving Entity pursuant to Step 3.  The Import Capacity allocated pursuant to Step 3 shall be referred to as “Commitment Import Capacity.”  This task has been completed.

- To the extent import capacity remains unallocated following Steps 1-3, the CAISO will publish on its website remaining aggregate import capacity, the identity of the branch groups with available capacity, and the MW quantity remaining on each such branch group.  The remaining aggregate import capacity will be allocated to Load Serving Entities serving load within the CAISO Control Area through their Scheduling Coordinators based on each Load Serving Entity’s Import Capacity Load Share.  The quantity of import capacity allocated to Load Serving Entity is that entity’s “Remainder Import Capacity.”  This Step 4 does not allocate import capacity on a specific branch group, but rather allocates aggregate import capacity.  This task has been completed and the information is available on the CAISO website at  http://www.caiso.com/1838/1838d8b963bb0.pdf.

- Load Serving Entities will have one week to trade some or all of their Remainder Import Capacity to any other Load Serving Entity or market participant.  The CAISO will accept trades among LSEs and market participants only to the extent such trades are reported to the CASIO as outlined in a CAISO Market Notice.   Import Capacity trading ended on July 26, 2006.

- By July 31, 2006, three business days after the close of trading as set forth in Step 5, Scheduling Coordinators for LSEs and other market participants shall report to the CAISO requests to allocate post-trading Remainder Import Capacity on a MW per available branch group basis.  The CAISO will honor the requests to the extent a branch group has not been over-requested.  If a branch group is over requested, the requests for Remainder Import Capacity on that branch group will be allocated based on the ratio of each Load Serving Entity’s Import Capacity Load Share, as used in Step 3.  A market participant without a Coincident Load Share will be assigned the Coincident Load Share equal to the average Coincident Load Share of those Load Serving Entities from which it received Remainder Import Capacity.  The CAISO will notify each Scheduling Coordinator for Load Serving Entities of their accepted allocations under this Step 6 on August 10, 2006.

- The CAISO will publish on its website on August 10, 2006 any remaining aggregate import capacity if any, the identity of the branch groups with available capacity, and the MW quantity remaining on each branch group.  To the extent import capacity remains unallocated, in the time period and manner established by CAISO market notice, all LSEs will notify the CAISO of their request to allocate any Remainder Import Capacity on a MW per available branch group basis by August 16, 2006.  The CAISO will honor the requests to the extent a branch group has not been over requested.  If a branch group is over requested, the requests on that branch group will be allocated based on the ratio of each Load Serving Entity or market participant’s Import Capacity Load Share, as used in step 3 and 6.  The CAISO will notify each Scheduling Coordinator for a Load Serving Entity of the Load Serving Entity’s accepted allocation under this Step 7 by August 23, 2006.  No further iterations will be permitted.

This multi-step allocation of import capacity does not guarantee or result in any actual transmission service being allocated and is only used for determining the maximum import capacity that can be credited towards satisfying a Load Serving Entity’s planning reserve margin, or appropriate Resource Adequacy Obligation.  Upon the request of the CAISO, Scheduling Coordinators must provide the CAISO with information on existing import contracts and any trades or sales of their load share allocation.    The CAISO will inform the CPUC or other Local Regulatory Authority of any Resource Adequacy Plan submitted by a Scheduling Coordinator for a Load Serving Entity under their respective jurisdiction that exceeds its allocation of import capacity.

Again, please refer to Appendix C of this Guide for a quick reference guide as to the timelines and tasks that are codified in Section 40.5.2.2 of the CAISO’s Tariff.

- DWR Contracts

All DWR CERS contracts that are not unit specific shall be entered on a separate worksheet page in the System RA Filing Template.  This page allows the LSE to enter DWR contracts that may or may not fit in the other pages of the template.  In order to facilitate evaluation of Local RAR procurement, please enter all unit specific DWR contracts and their CAISO IDs on the Physical Resource page, and enter the Local Area designation of the unit.  The Local Area designation is taken from the CAISO Local Area Units List, while the correct CAISO ID and MW QC total both come from the CAISO QC list.

- Liquidated Damages (LD) Contracts

Liquidated damage (LD) contracts do not count towards fulfillment of Local RAR, due to their nature as non-unit specific.  Additionally, in order to be counted for the 2007 System RAR Filing, LD contracts must comply with the following rules:

- No LD contract entered into after October 27, 2005 can be used for RA purposes.

- LD contracts entered into on or before October 27, 2005 can not be modified to increase the number of MW or the duration of the contract for RA purposes.

For all filings applicable to 2007, LD contracts cannot count for more than 50% of the LSE’s RAR portfolio.  This phase out is reflected in the last line of Summary Table 6 on the template.  LD contract MW in excess of 50% of an LSE’s RAR portfolio do not count for RA compliance.  For example, if LSE-1’s total RAR is 1,000 MW, then LSE-1 cannot count more than 500 MW of LD contracts towards its RA filing requirement. The LSE must show that at least 50% of their RAR is met through non-LD contract resources.

Firm import LD contracts with specific Intertie Points and Import Allocations are not subject to these phase-out provisions and should not be included on the LD page, but listed instead on Worksheet III Non-Unit Contingent Import.

To the extent that procurement meant to satisfy Local RAR and the sum of the rest of the LSEs procurement causes overprocurement, and in effect restricts the counting of LD contracts further than the phase out percentage would otherwise accomplish, the Local RAR takes precedence.

- Resource Adequacy (RA) Portfolio Resources

D.06-07-031 determined that the RA compliance process can accommodate the limited use of Resource Adequacy (RA) Portfolios, which are plant-specific RA contracts, not unit-specific RA contracts.  For example, for the System year-ahead compliance filing, LSE-ABC may want to enter into an RA contract with a generator for 200 MW to be provided by any one of three units at the Acme Power Plant, as opposed to a specific unit.

For purposes of the year-ahead 90% compliance filings only, RA portfolios are acceptable, subject to the following conditions:

- The portfolio must be eliminated and converted to specific units in the Monthly System RA showing.

- Any portfolio must be unique and the units behind that portfolio must be specified, communicated to the CAISO/CPUC, and not allowed to change.

- The portfolio may not have total capacity greater than the summed Qualifying Capacity of the individual units.

- Portfolio may only be comprised of units served by the same busbar.

- All units are located in the CAISO control area.

The 2007 Local RA compliance showings do not allow the use of portfolio resources.

- Certification of LSE Resource Adequacy Compliance Filing

As confirmed in D. 06-07-031 all RA Filings shall be filed under the following certification, and a certification sheet must accompany each template.

Consistent with Rules 1 and 2.4 of the CPUC's Rules of Practice and Procedure, this Resource Adequacy compliance filing has been verified by an officer of the corporation who shall expressly certify, under penalty of perjury, the following:

- I have responsibility for the activities reflected in this filing;

- I have reviewed, or have caused to be reviewed, this compliance filing;

- Based on my knowledge, information, or belief, this filing does not contain any untrue statement of a material fact or omit to state a material fact necessary to make the statements true;

- Based on my knowledge, information, or belief, this [filing] contains all of the information required to be provided by Commission orders, rules, and regulations.

The actual certification page that must be completed and signed is included as a separate tab in each of the template workbooks.

- RA Filings Instructions

The 2007 Local RA Filings cover Local RAR for the entire 2007 compliance year, and the System RA Filings cover System RAR for the months of May through September 2007.  These filings are due at the Commission Office and at the CEC and ISO offices, according to the detailed schedule below.  Postmarks will not be accepted.  Pursuant to D. 05-10-042, filings are to be made via Advice Letter.  Instructions for filing Advice Letters for RA compliance follow in Appendix A, and an Advice Letter Summary Sheet to be included in the RA Filing follows in Appendix B

| - Type of Filing | - Due Date |
| --- | --- |
| - Preliminary Local and System RAR Year Ahead | - September 22, 2006 |
| - Final Local and System Year Ahead | - October 31, 2006 |
| - CAISO notifies all LSEs of any collective deficiencies to allow for additional LSE procurement. | - November 8, 2006 |
| - January Month Ahead | - November 30, 2006 |
| - Last date for LSE to file amended Local RAR or System “year ahead” RAR showing to reduce CAISO backstop for collective deficiency. | - December 1, 2006 |

File with the agencies at the addresses below:

- An Advice Letter that complies with the rules in Appendix A and includes a Summary Sheet;

- Completed workbooks covering System RAR for the summer months of May through September and the Local Template for all months of the year

| CPUC Energy Division 505 Van Ness Avenue 4th Floor San Francisco, CA 94102 Attn: Tariff Room (RAR Filing) Please include 3 (or 2 if no returned copy is needed) hardcopies and 1 electronic copy on CDROM. | California Energy Commission 1516 Ninth Street Sacramento, CA 95814 Attn: Mike Jaske (RAR Filing) Please include 1 electronic copy on CDROM. | CAISO 151 Blue Ravine Road Folsom, CA 95630 Attn: RAR Filing Please include 1 electronic copy on CDROM. |
| --- | --- | --- |

TWO hardcopies and one electronic copy (by CDROM in Word and Excel format) of the entire RA Filing (Advice Letter with Summary Sheet and RA Templates) shall be submitted to the CPUC Energy Division at the address above.  The entire RA Filing is also to be submitted electronically to the CEC and CAISO (by CDROM in Word and Excel format) at the addresses above.  If an LSE would like the CPUC to return one paper copy of the filing stamped and dated as received, the LSE must so request in the advice letter and submit an extra hardcopy.  A release form is available to an LSE that wishes to submit electronic copies via email instead of CDROM.  The release is to be signed by the same officer as the certification page.  The Energy Division will report Advice Letters and related documents, and the date of their receipt, in the Daily Calendar.

We have included one page for each worksheet.  If more are needed, add rows as needed to the Excel spreadsheet.  It is the responsibility of the LSE to ensure that all formulas used in the spreadsheet are not changed, and that all information is integrated into the formulas correctly.  We specifically note that the formulas in the Resource Adequacy Capacity column of the resource worksheets should not be changed.  In addition, any cell that is not colored blue on the Summary Worksheet of the RA Template is not to be changed.  Please do not print out and mail this RA Guide or the template instructions.  You can remove these from your filing, as they are unneeded for compliance review.  LSEs are also encouraged to print double sided whenever possible both to conserve paper and to reduce the mass of the filings.

- Correction of Errors: Minor or Substantial

There are specific Advice Letter rules and procedures LSEs must follow when making corrections or addition to their RA Filings.  These are detailed in Appendix A, Primer for the Filing of Advice Letters.  There are three classes of corrections:  those that require replacement sheets to correct typos and numerical errors, minor errors that require Supplemental Advice Letter filings which clearly explain the corrections, and more substantial errors which require the filing of an entirely new Advice Letter.  The CPUC has discretion over classifying errors, and ordering corrections.

APPENDIX A

Submission of Advice Letters for Resource Adequacy Compliance Filings

1. Applicability

D.05-10-042 ordered LSEs to make RAR compliance filings via the Advice Letter process, including Annual Local, Annual System, and Monthly RA Filings.

1.1 Code of Ethics

Rule 1 (“Code of Ethics”) of the Commission’s Rules of Practice and Procedure (California Code of Regulations, Title 20, Division 1, Chapter 1) shall apply to all RA Filings submitted as Advice Letters.

1.2 Computation of Time

As used in these rules, “day” means a calendar day, and “business day” means a calendar day except for Saturdays, Sundays, and weekdays when the Commission’s offices are closed, due either to a State holiday or to an unscheduled closure (e.g., an emergency or natural disaster).  The Commission’s Internet site (www.cpuc.ca.gov, under “About CPUC”) will maintain a list of State holidays for the current calendar year and a list for the following calendar year as soon as that list is available.

When these rules set a time limit for performance of an act, the time is computed by excluding the first day (i.e., the day of the act or event from which the designated time begins to run) and including the last day.  If the last day does not fall on a business day, the time limit is extended to include the first business day thereafter.

2.  Cover Sheet

The RA Filings (Advice Letter with Summary Sheet and all RA Templates) shall include a cover letter, which shall state the date when the LSE served the Advice Letter and filed it with the Energy Division.  A Summary Sheet is attached as Appendix B.  The cover letter shall summarize the contents of the Advice Letter, as follows:

- Indicate requested effective date;

- Indicate service affected by the Advice Letter (in this case Retail Electric Service);

- Cite Commission orders by decision or resolution number (in this case D.05-10-042 and D.06-06-064) and Public Utilities Code or other statutory provisions (by section) related to the substance of the Advice Letter, and identify as compliance filing (where applicable);

- Indicate whether the LSE believes disposition of the Advice Letter by Commission resolution is necessary or appropriate (RA compliance filings do not require a resolution);

- Show contact person, telephone number, and e-mail address for additional information regarding the Advice Letter

- Protest Period; state that RA compliance filings are not subject to protest.

If an Advice Letter does not include a complete cover letter, as described above, the Energy Division may reject the Advice Letter without prejudice or extend the protest period unless and until the LSE files the information that is missing or incomplete with the CPUC.

3.  Form and Content

An LSE shall:

(1)  number the Advice Letter sequentially, beginning with No. 1 for the first Advice Letter filed by the LSE, followed by the letters ESP if the LSE is an ESP or E if the LSE is an IOU.

(2)  state whether any deviations would be created, service withdrawn from any present customer, or more or less restrictive conditions imposed;

(3)  state the notice requirements applicable to the Advice Letter, and describe how those requirements were satisfied.

4. Filing Advice Letters and Related Documents

The RA filing (Advice Letter with Summary Sheet, and all applicable RA Templates) shall be submitted to the CPUC Energy Division, CEC, and CAISO.  The number of copies and method of filing are summarized in Section 12 of the RA Guide, along with the exact mailing addresses to be used at the CPUC, CEC, and CAISO.

5. Confidentiality

Pursuant to the February 10th Revised Protective Order and D. 05-10-042, the entire RA filing (the Advice Letter, Summary Sheet, RA template, and LD template) is protected and confidential.  Each page of each document should be marked, "Protected Materials," "Protective Order," or "Section 583" and sequentially numbered (e.g. Bates stamped).  For digital materials, each disk or other storage device should also be so marked.

6. Service to Other Parties

The System and Local 2007 RA filings are compliance filings, confidential, and not subject to protest. Therefore, service beyond the parties listed in Section 12 of the RA Guide (CPUC, CEC, and CAISO) is not required.

- Correction of Errors made in RA Filings

Corrections to the RA Filing are to be made according to the rules for correcting errors in Advice Letters as established in GO 96-A.  Minor typographical or numerical inaccuracies can be made by submitting corrected pages to replace incorrect pages. The LSE must type REVISED at the top of all corrected pages.   Other minor errors can be corrected by submitting a Supplemental Advice Letter that details minor changes.  Major errors may require the filing of an entire replacement RA Filing with advice letter.  The PUC Staff reserves the discretion to classify errors as one of the three classes, and to order corrections.  Directions for all three cases follow.

- Typographical and Numerical Errors:

Simple typographical or numerical errors can be corrected by the LSE or upon request by the PUC by submitting revised pages to replace the pages with the errors.  Service of revised pages must be made on paper, with the intention and ability to easily swap the new pages into place without inconsistencies.  Along with service on paper, the LSE is to file corrections electronically, and include the entire file with the corrections integrated by CDROM in Word and Excel format.  Corrections are to be mailed to the CPUC at the address in section 12 of the RA Filing Guide.  Once the filing is corrected and approved, the CPUC will mail a final edition of the filing to the CAISO and CEC on CDROM.  Corrections of this type do not require an Advice Letter Cover Sheet or Summary Sheet.  Examples of typographical or numerical errors include typing one digit incorrectly in a Scheduling ID, filing a number 1243 for 1234, etc.

Correction and submittal of corrected pages must be completed within five business days after notification by the CPUC Energy Division via telephone or email.

- Other Minor Errors

Other minor errors must be corrected through the filing of a Supplemental Advice Letter, with the corrections clearly explained in the Advice Letter Cover Sheet and noted on the Summary Sheet in Appendix B.  The numbering of the Advice Letter is to be the same as the original Filing, with the addition of the letter ‘A’ to the end of the number (e.g. 1ESP-A).  Subsequent corrections to the same original Advice Letter should use subsequent letters.  For example, the second correction of the original letter detailed above would hold the number 1ESP-B.

Supplemental Advice Letters are evaluated similarly to regular Advice Letters, and are subject to the same filing provisions.  Examples of minor errors include omitting resource availability, filing a resource under an incorrect tab (recording an LD contract as an import), and any typographical or numerical error that would change an LSE’s compliance status.  The Supplemental Advice Letter is to be mailed to the CPUC at the address in section 12 of the RA Filing Guide.  Once the filing is corrected and approved, the CPUC will mail a final edition of the filing to the CAISO and CEC on CDROM.

Correction and submittal of corrected pages must be completed within five business days after notification by the CPUC Energy Division via telephone or email.

- Substantial Revisions – Entirely New Advice Letter

For more substantial errors, the LSE may be asked to refile a second complete RA Filing including RA Filing workbooks and advice letter with Summary Sheet.  The numbering of the Advice Letter will use the next available number, and list the original Advice Letter that is being replaced on the appropriate spaces on the Summary Sheet.  The Advice Letter Cover Sheet should also clearly explain the reasons behind the second filing of the entire RA Filing.  The new Advice Letter is to be mailed to the CPUC, CEC, and CAISO at the addresses in section 12 of the Filing Guide.

CALIFORNIA PUBLIC UTILITIES COMMISSION

ADVICE LETTER FILING SUMMARY

ENERGY UTILITY

| MUST BE COMPLETED BY UTILITY (Attach additional pages as needed) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Company name/CPUC Utility No. |  |  |  |  |  |  |  |
| Utility type: |  |  | Contact Person: _______________________________ |  |  |  |  |
|  ELC |  GAS |  | Phone #: |  |  |  |  |
| ¨ PLC | ¨ HEAT | ¨ WATER | E-mail: |  |  |  |  |
| EXPLANATION OF UTILITY TYPE ELC = Electric GAS = Gas PLC = Pipeline HEAT = Heat WATER = Water |  |  |  |  | (Date Filed/ Received Stamp by CPUC) |  |  |
| Advice Letter (AL) #: |  |  |  |  |  |  |  |
| Subject of AL: Keywords (choose from CPUC listing): |  |  |  |  |  |  |  |
| AL filing type: ¨ Monthly ¨ Quarterly ¨ Annual  One-Time ¨ Other _____________________________ If AL filed in compliance with a Commission order, indicate relevant Decision/Resolution: Does AL replace a withdrawn or rejected AL? If so, identify the prior AL ____________________________ Summarize differences between the AL and the prior withdrawn or rejected AL1: ____________________ Resolution Required? ¨ Yes  No |  |  |  |  |  |  |  |
| Requested effective date: |  |  |  |  |  | No. of tariff sheets: |  |
| Estimated system annual revenue effect: (%): Estimated system average rate effect (%): When rates are affected by AL, include attachment in AL showing average rate effects on customer classes (residential, small commercial, large C/I, agricultural, lighting). |  |  |  |  |  |  |  |
| Tariff schedules affected: Service affected and changes proposed: |  |  |  |  |  |  |  |
| Pending advice letters that revise the same tariff sheets: |  |  |  |  |  |  |  |
| Protests and all other correspondence regarding this AL are due no later than 20 days after the date of this filing, unless otherwise authorized by the Commission, and shall be sent to: |  |  |  |  |  |  |  |
| CPUC, Energy Division Utility Info (including e-mail) |  |  |  |  |  |  |  |
| Attention: Tariff Unit 505 Van Ness Ave., San Francisco, CA 94102 jjr@cpuc.ca.gov and jnj@cpuc.ca.gov |  |  |  |  |  |  |  |

Appendix C

California Independent

System Operator Corporation

| CAISO Tariff Section 40.5.2.2 – Deliverability of Imports |  |  | SC/LSE OBLIGATIONS |  | CAISO OBLIGATIONS |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tariff Step | Task No. | Required Tasks by Step | Due Date | Status | Due Date | Status |
| 1 | 1 | Publish total import capacity values on CAISO website |  |  | 1-Jul | Complete |
| 2 | 2 | Publish on the CAISO website the total import capacity minus existing transmission contracts, encumbrances, and transmission ownership rights |  |  | 1-Jul | Complete |
| 3 | 3 | CAISO receives from LSEs resource commitments from resources outside the CAISO | June | Complete |  |  |
| 3a | 4 | Receive Coincident Load Share information from CEC | 11-Jul | Complete |  |  |
| 3b | 5 | Notify the SC for each LSE of the LSE’s coincident load share |  |  | 19-Jul | Complete |
|  | 6 | Notify the SC for each LSE of the amount of import capacity by branch group that has been allocated to the LSE pursuant to Step 3 |  |  | 19-Jul | Complete |
| 4 | 7 | Publish remaining aggregate import capacity and the available MWs on the branch groups on the CAISO website |  |  | 19-Jul | Complete |
| 5 | 8 | Notify the SC for each LSE of the LSE’s share of remaining import capacity; (not branch group specific) |  |  | 19-Jul | Complete |
|  | 9 | Market notice issued that sets the date import capacity trading closes for Step 5 and 6 when/how LSEs should notify the CAISO of their trades and how they shall request capacity in Step 7 |  |  | 19-Jul | Complete |
|  | 10 | Import capacity trading ends and LSEs report trades of import capacity to the CAISO | 26-Jul | Complete |  |  |
| 6 | 11 | LSEs request allocations of import capacity by branch group. Total requests by the SC for the LSE cannot exceed its post-trading Remainder Import Capacity amount | 31-Jul | Complete |  |  |
|  | 12 | CAISO will notify the SC for each LSE of the LSE’s import capacity allocation |  |  | 10-Aug | Complete |
| 7 | 13 | Publish remaining aggregate import capacity, if any, and the available MWs on the branch groups on the CAISO website |  |  | 10-Aug | Pending |
|  | 14 | LSEs request allocations of import capacity by branch group. Total requests by the SC for the LSE cannot exceed its Remainder Import Capacity amount following Step 6 | 16-Aug | Pending |  |  |
|  | 15 | CAISO notifies the SC for each LSE of the LSE’s import capacity allocation |  |  | 23-Aug | Pending |

Appendix D

Frequently asked questions and clarifications to the filing instructions:

1.  Question:  What if I have more than one contract with the same facility?  Maybe there is a baseload contract with a generator for part of the capacity, but also peak capacity contract for the rest? How should I file that in the RA template; should I include all that information in one line with one contract ID?

Answer: Please list each separate contract on separate lines consecutively in the RA template.  The Scheduling Resource ID (column C) will remain the same, but the Resource contract number (column G) will be different.  Please list all information for each contract because functionally they are different contracts.

- Question:  What if I have one contract for peak capacity for 15 MW and a second contract for off-peak capacity for 5 MW?  How should I report these contracts in the RA template; should I include all that information in one line with one contract ID?

Answer:  If the peak and off peak contracts combine to cover a 24 x 7 period, split the peak contract into two components; 5 MW to match with the off-peak contract and 10 MW that remain peak.  Then, on one line report the 5 MW peak and 5 MW off peak contracts as a single resource in category 4 with unrestricted availability (all hours).  On a second line report a 10 MW peak contract.  On the line with two contracts, both contract numbers should appear in the contract ID cells.

- Question:  What if I have one contract with a facility, or an LD contract, that includes different components?  For example 100 MW 7x24, and 15 MW 7x16? How should I report that in the RA template; should I include all that information in one line with one contract ID?

Answer:  If a single resource contract has separate components that qualify in different resource categories, the contract should be entered in the RA Template in multiple lines.  Using the example, one line should be completed using the 100 MW 7x24 component and a separate line should be completed using the 15 MW 7x16 component.  Each line should include all information.

4.  Question:  What does it mean in the instructions for Minimum Hours in Month, where the directions refer to “during peak load hours?”

“Minimum Hours in Month - The minimum number of hours in the RA month that the RA resource is contractually or physically available and capable of operating at its Qualifying Capacity during peak load hours to meet the LSE’s RAR.”

Answer:  The minimum hours in a month are the minimum hours that a resource is available.  For example a 5x4 contract is available for 80 hours a month.  To count, those hours must be peak hours.  A 5x4 contract that is available between 2 and 6 am would not deliver RA benefits.  Different programs have different definitions of peak hours, so for this template peak hours are counted in accordance with program rules.  For example, solar and wind resources define peak as noon to 6pm per D 05-10-042.

5.  Question:  Do firm import LD contracts signed after October 27, 2005 still count towards RA requirements, or are they subject to the same sunset date and phase out percentages as in-area LD contracts are pursuant to page 65 of D. 05-10-042?

Answer:  Firm import LD contracts do not fall under the sunset and phase out provisions because they do not present the same deliverability and reliability issues as in-area LD contracts.  Thus Firm import LD contracts with specific intertie agreements do not fall under the same phase out schedule.

6. Question:  What is the difference between Scheduling Resource ID in Column C and the Resource Contract Number in column G in Worksheets I, II, and IV in the RA Template?

Scheduling Resource ID – The CAISO-assigned Scheduling Resource ID.

Resource Contract Number – LSE specified number that identifies the relevant contract(s).  This information will be used to identify supporting documentation during compliance verification.

Answer:  Column C, Scheduling ID in CAISO Master File is a standard ID name supplied for a particular unit from the CAISO Master File.  Column G, Resource Contract Number, refers to the LSE’s contract ID number.  If there are two contracts with the same unit, then contract numbers (column G) would be different, but the Scheduling Resource ID or CPUC LD contract number (Column C) would be the same. Please refer to Question 1 above.

7.  Question: The RA guide tells an LSE to input into separate DR worksheets those programs that are over 2 hours per day and those that are under 2 hours per day.  The guide also states that the total amount of DR Allocation is to be found in a letter received from the CEC.  This letter from the CEC does not make clear which programs are under 2 hours a day and which are over 2 hours a day; the letter only gives total capacity.  Where do I enter this allocation?

Answer:  All the programs listed in the CEC letter are for over 2 hours a day; that was how the CEC analyzed it.  Put all programs listed on the CEC letter for over 2 hours a day.

8.  Question:  What is the desired level of accuracy for numbers, 1 MW, 0.5 MW, 0.01 MW?  Do I round numbers off to the nearest 1MW, 0.5MW or 0.01 MW?

Answer:  For purposes of filling in the templates, round off to the nearest 0.01 MW like the load forecasts and RMR allocations do.  Do not round any more than that.  Precision is important, particularly for smaller LSEs

9.  Question:  If I have no LD contracts, do I need to file the LD worksheet in the RA Template?

Answer:  No.  Just state it in the Advice Letter that the LD worksheet is unneeded. In fact, any part that does not apply to the LSE can be left out, such as DWR contracts, with a statement in the Advice Letter stating which parts are missing and why.  Please also limit submissions of blank pages.  Just use the sheets that you need.  Also the PUC encourages the LSEs to print double sided whenever convenient, to conserve paper and reduce the mass of the filings.  It is the responsibility of the LSE, however, to ensure that no formulas on the Summary Page are damaged by the removal of worksheets, as summary data is drawn from all parts of the template.

- Question:  What if I have a contract with a unit that lasts for only part of the month?

Answer: Please pair up the resource with another resource that can fill out the month as done for peak/off peak pairings in question 2 above.  If that is impossible, a contract for part of a month will not count for RA and should not be listed.

Appendix 1 deals with IOU data, and Appendix 2 deals with ESP data.

Note a revision was published on August 9, 2006.

Pursuant to Commission Decision 06-07-031 section 4.1 and 4.2

The CEC mailed out 2007 DR allocations and load forecasts by certified mail on July 12, 2006.

The list will be available from the CAISO prior to the Filing Date.

Does not include monthly compliance filings for System RAR

Both in R.05-12-013

Discuss in AL if more space is needed.