____________________________________

Order Instituting Rulemaking on the		)

Commission’s Proposed Policies		)

Governing Restructuring California’s   	)	R. 94-04-031

Electric Services Industry and		)

Reforming Regulation				)

____________________________________)

Order Instituting Investigation on		)

the Commission’s Proposed Policies		)

Governing Restructuring California’s		)	I. 94-04-032

Electric Services Industry and		)

Reforming Regulation				)

)

____________________________________)

PG&E STATUS REPORT ON DYNAMIC LOAD PROFILING

I. INTRODUCTION

PG&E submits this Dynamic Load Profiling Status Report (Report) pursuant to the Commission’s instructions in the October 30, 1997 Load Profile Decision (D. 97-10-086):

PG&E and SDG&E shall file with the Docket Office and serve a status report within 30 days from today’s date as to the reasons why dynamic load profiling cannot be instituted for their residential and small commercial and industrial customers by January 1, 1998.

(1)  Such a status report shall also inform the Commission as to what steps they will take to ensure that dynamic load profiling can be in place beginning on July 1, 1998, as well as the estimate of the costs that they anticipate spending to institute [dynamic] load profiles.

(D. 97-10-086, Ordering Paragraph 2.b)

PG&E continues to support, as it did in the original Load Profile Workshop, the move towards more accurate load profile methodologies, including the development of dynamic profiling techniques.  Greater accuracy in profile estimation will reduce cost-shifting among customers and energy suppliers associated with the misallocation of hourly generation responsibility, and will reduce the amount of unaccounted for energy imbalance that must be allocated administratively by the ISO, improving the overall economic efficiency of the direct access marketplace.

Dynamic profiling has the potential to improve accuracy over the interim static methodology because it incorporates real weather and usage information from the profile day into the estimation of a statistical load profile.  However, this potential improvement in accuracy will only be fully realized if there is sufficient time available to perform validation, editing and estimation (VEE) processes -- essential data validation checks and quality assurance (such as re-polling of non-responding sample meters) -- and to process sample data into statistically valid class-average load profiles, prior to the publication of a final, dynamic load shape for a given profile day.  PG&E believes, without sufficient data processing time (and assuming only existing technology), dynamic profiling may provide little or no improvement in accuracy over the interim static approach.  In fact, there is even the possibility of a loss of accuracy relative to the interim static approach if insufficient time is allowed for VEE processes.  PG&E’s dynamic implementation therefore will allow a processing lag of one week to ensure that dynamic profiles will be an improvement over static ones. This lag is discussed further in Section III. E. below.

As the Commission noted in its Load Profile Decision, agricultural customers, due to “their varying sizes, their locations, and changing usage” (D. 97-10-086, p.17), may be less cost-effective candidates for dynamic sampling than other customer classes.  The applicability of dynamic methods for agricultural customers is the subject of a Commission ordered workshop to be held by February 15, 1998.  So as not to pre-judge the issues in this workshop, PG&E does not evaluate either the costs or feasibility of dynamic profiling for agricultural customers in this Report.

Finally, PG&E believes that the Commission intended dynamic profiling to replace the interim static methodology on July 1, 1998, for all applications of load profiling under its jurisdiction, for the applicable customer classes (i.e. all customer classes other than streetlights, traffic controls, and possibly agricultural customers).  Pursuant to the Cost Separation Decision (D. 97-08-056), PG&E will be required to employ a class-average load profile in determining the non-energy charges for bundled service, VDA, and Direct Access customers.  This calculation will require the derivation of class load profiles even for those customer classes above 50 kW maximum monthly demand, who are not otherwise eligible to take Direct Access using a statistical load profile. PG&E therefore includes in its dynamic implementation plan the  development of dynamic samples for all residential, commercial, and industrial customer classes, including its large commercial and industrial classes (i.e. PG&E rate schedules E-19,  E-20, and Standby).

In Section II of this Report, PG&E explains how dynamic load profiling is dramatically different from current load research practice, and why the switch to a dynamic methodology will not be achieved by January 1, 1998.  Section III then lays out PG&E’s plan for implementing dynamic load profiling by July 1, 1998, and estimates the costs and resource requirements associated with this implementation.   Finally, Section IV briefly summarizes the policy tradeoff between data accuracy and timeliness of profile distribution, which PG&E believes will be an important issue for further discussion among parties during 1998.

II. IMPEDIMENTS TO THE IMMEDIATE IMPLEMENTATION OF DYNAMIC LOAD PROFILING

The Load Profile Decision states:

The Workshop Report and the filed comments did not shed any light on why it is not feasible for PG&E and SDG&E to institute dynamic load profiling before January 1, 1998. We assume that part of the problem is that they may not have enough load research meters in place to create dynamic load profiles. (D. 97-10-087, p. 17)

An increased sample meter requirement is indeed a  piece of the challenge PG&E faces in implementing dynamic load profiling in the coming months. However, there are other impediments as well.  Figure 1 shows the various steps involved in PG&E’s business process for class load estimation/load profiling.  Dynamic profiling will require significant modifications to every one of the steps in order to accommodate the order of magnitude increase in data volumes and  turn-around times necessitated by the dynamic approach. These changes are described in detail in Section III.

Dynamic load profiling is dramatically different from traditional load research procedures because it requires the daily retrieval of data from  sample meters, the daily validation of that data, and the daily calculation of new, statistically representative class-average load profiles.  None of this work has traditionally been performed on a daily basis. Currently, PG&E’s load research is designed to support annual class load shape estimation of prior-year hourly and half hourly load patterns for major customer classes and subgroups.  This information is used periodically in revenue requirements analysis, rate design, and regulatory reporting.  While the current process retrieves sample interval data and estimates class load for a specific day based on information from that day, the use of that information has not been required on a next-day or even next-week basis. In particular, the “dynamic” use of class load research data has never before been required for either customer billing or financial settlements transactions.

Today’s load research sample design, data retrieval systems, data validation processes, and load shape estimation are all designed to create new  class load shapes only once per year.  Current rates are based on historical and forecasted average load shapes.  Discrepancies between time-of-use dependent system costs and revenues collected through rates can then be adjusted retrospectively, through the ratesetting process.  In order to support billing and settlement activities based on daily, dynamic load profiles, the time frame for the entire load research class load estimation process must be compressed, from an annual analysis time-frame, to a continuous, automated daily process.

PG&E will not be able to implement dynamic load profiling by January 1, 1998, because of the magnitude of the changes described above and detailed in Section III Furthermore, PG&E load research staff have devoted nearly all their time since June of this year, developing and implementing the static load profiling process, approved for interim use on January 1, and therefore have not been available to make further refinements to the load profiling process.  PG&E expects to be able to complete implementation of dynamic profiling for its residential, commercial and industrial customer classes by July 1, 1998.

The following section of this Report details PG&E’s business process for load profiling step by step and describes  the changes to current business practice that are necessary to implement the dynamic approach by July 1.

III. PG&E’S PLAN FOR IMPLEMENTATION OF DYNAMIC LOAD PROFILING BY JULY 1, 1998

PG&E’s dynamic implementation will require major changes in current load research practice.  These changes are described in detail below.  The costs of PG&E’s dynamic profile implementation are shown in Table 1.  PG&E estimates the total costs of its implementation to be approximately $3 million in one-time costs, with recurring annual costs of an additional $2-3 million.  Recurring costs are primarily labor-related expenses, of maintaining the data acquisition and VEE processes.

A. Statistical Sample Design

The load estimation process begins with the development of a sampling plan and determination of the appropriate, statistically representative load research customer sample in each rate class and stratum.    Weights are derived for each sample stratum and applied to sample means by stratum.  These weighted means are then summed to produce preliminary class estimates.  The class consumption figures obtained from billing data are then applied to the preliminary estimates to produce final class estimates, known as “ratio estimates”.  The accuracy of the sample is evaluated taking into account the statistical nature of these ratio estimates.  Pursuant to PURPA standards, statistical accuracy for major classes is held to within 10% of true value at time of system peak, with 90% confidence.

Certain sample changes will be necessary to retain class load accuracy at current levels.  First, in order to compensate for the potential data retrieval problems inherent in an increased reliance on telephone modem communications, PG&E proposes to supplement the current residential and commercial sample by 10% (discussed further in  III B below).  PG&E believes this increase is a minimum necessary for July 1. Further experience and evaluation after July 1, may lead to additional increases in sample size.  In addition, time of use (TOU) residential and commercial customers will require additional sample points to allow for the modified process required to construct profiles for these rate classes. Finally, an automated system for tracking sample customer migration to direct access and non-UDC meter data management provision will be needed, to ensure continued daily access to interval data for all dynamic sample meters (see further discussion below under III C).

B. Field Equipment Installations

The standard load research equipment installation at PG&E currently consists of a revenue meter retrofitted with a pulse initiating device, commonly referred to as a PI meter, and a stand alone interval data recorder.  Most, but not all, recorders contain modems.  Load data is then retrieved via phone line or manually, via a hand held device.

The data retrieval and translation system is the proprietary MV90 protocol, from Utility Translation Systems (UTS). Currently, approximately 700 of the 3400 sample points representing residential, small and medium commercial, and agricultural classes are read with hand held devices monthly.  Over 75% of these are in the agricultural sector.

Currently, even those meters with phone lines only have the interval data retrieved on a weekly or biweekly schedule.  Of the phone line retrieval sites, at least 10% fail to connect and/or achieve complete retrieval by the MV90 system on the initial attempt overnight (which includes up to 3 attempts to call). In addition, 10-20% of retrieved interval data reads currently fail one or more validation checks (see section III. D.), which may result in additional retrieval attempts.

The move to dynamic profiling will lead to significantly higher volumes of overnight retrievals, and the loss of any ex post manual interrogation capability, and will therefore likely result in lower overall capture rates for sample data, on a daily basis.  PG&E proposes at least a 10% increase in its residential and small commercial samples (400  meter sets) to compensate for this anticipated loss of data availability.  PG&E believes this change is necessary, in order to maintain the accuracy of load profiling at current high levels.

Finally, in order to have a fully dynamic sample group, an estimated 200 residential and commercial sites will require new phone line installations for data retrieval.  One hundred of these 200 sites, as well as an additional 500 sites, will need the recorder equipment changed out, for a total of 600 retrofits.  Load research has ten different types of recorder equipment in the field.  The sites identified above represent several older models which have been identified as experiencing increased communication failure rates and will need to be replaced to increase the probability of daily interval data retrieval.

The total numbers of meters requiring replacement, retrofit, and phone line installation, by category, along with the anticipated costs of these changes, are shown in Table 1.

C. Data Acquisition

Data acquisition is the process of (primarily) phone-line based retrieval of raw interval data from the load research sample recorders, including repeated attempts to capture data from non-responding sample points.  The current data acquisition process will require significant enhancements to provide for daily retrieval of load research sample information, such as upgrades or expansions to modem and data processing capacity, and automation of manual quality assurance processes.

The data acquisitions systems currently in place were not designed for daily data retrieval. This work will entail extensive software and hardware upgrades and new personnel.  PG&E is currently exploring both in-house and outsourcing options for improving data acquisitions for dynamic load profiling.  In addition to these systems changes, the unbundling of the meter data management function will likely require PG&E systems to accommodate alternate sources of load research information (other than direct connection to the recorder/meter by PG&E), necessitating further data acquisition system enhancements.

D. Validation, Editing, and Estimation (VEE)

Raw sample meter data, like raw DA meter data, contains data gaps, outliers, and other erroneous or incomplete data that must be screened and corrected.  The current load research validation, editing, and estimation (VEE) process is similar to the proposed methods for direct access VEE outlined in the Metering PD, with two major differences.  First, the current process assumes the availability of complete billing information (i.e. monthly cumulating revenue meter reads and customer counts) for both the sample and total population.  In contrast, under dynamic profiling, complete billing information will not be available in time to compare load profile sample customer usage or counts of customers against PG&E system totals.

Current load research methodology relies on the availability of an independent revenue meter read for validation of the interval values.  Load research sites have their cumulative monthly energy usage determined from manual reads by meter readers during the normal billing cycle.  This information is then retrieved periodically during the year by Load Research personnel from the Customer Information System (CIS) and compared to summed interval values for the same time periods.  Problem reports, often resulting in field inspections, are issued for sites failing this cumulative energy consumption comparison.  Some problems can be resolved in ways that accept or correct the interval data, while others result in the deletion of erroneous interval values.  At the time of validation for analysis, at the end of the first quarter of the following calendar year, both monthly and annual energy comparisons are made and sites are wholly included or excluded from the analysis based on the comparison.  On average, 5-10% of the sample sites are rejected for analysis.

The second departure from the  VEE process proposed in the Metering PD for direct access is in the estimation process for missing data.  Individual load research site interval information is not estimated.  Missing data can simply be excluded from the analysis.  A minimum sample size per strata is required and strata are collapsed if necessary to preserve statistical validity.  Sample averages and weights for each sample point are calculated within strata prior to aggregation to a class average.

PG&E proposes that, as part of its dynamic implementation, it will develop a statistically valid dynamic VEE process, consistent with that in the Metering PD, including a default back-up estimation procedure for estimating profiles when an insufficient amount of sample data for a given class and day is available.  A threshold would be established at the profile level, consisting of a minimum number of sample sites which must be retrieved and pass basic validation within a 48 hour period.  The threshold verification process would be automated and included in the VEE software development.  Such a back-up would help to ensure the continuous flow of critical load profile data, in the event of any telecommunications mass failure (e.g. due to a catastrophic event).

Candidates for a back-up system include continued maintenance of static profiles beyond July 1, 1998, development of a VEE process system based on MADAWG-sanctioned data fill-in rules used for billing data, or the development of partially dynamic (e.g. system load-adjusted or historical “proxy day” profiles).  PG&E requires more time to investigate an appropriate methodology, but expects to have a back-up system in place by July 1.

E. Dynamic Profile Estimation

As discussed in Section I above, PG&E proposes a one week lag between the profile day and the publication of finalized dynamic load profiles to an Internet website, for use in billing and settlement applications.  PG&E’s assumption of a one week lag is entirely consistent with the definition of dynamic load profiling adopted by the Commission and is essential to allowing a reasonably thorough VEE process  to realize the greater accuracy potential of the dynamic method.  One week is synchronous with the timing of the rolling-weekly CTC calculation methodology, approved for use by PG&E, and will not further delay the ISO/PX settlement application of load profiles. It also preserves a matching of day-type between the profile day and the first calendar day for which profiles will be applied to eligible customer bills (i.e., Monday to Monday, Tuesday to Tuesday), which is convenient for the billing application.

The current load estimation system is programmed in Statistical Analysis Software (SAS) on the mainframe and will need to be completely overhauled to allow for this process.  As discussed in Section III. A. above, the current sample expansion process is a ratio method.  A simple mean-per-unit will not yield the same accuracy with current sample sizes.  A modified ratio technique will need to be developed.  PG&E is currently investigating alternative estimation software and techniques that would integrate with a VEE process and the default back-up profiling system.

F. Data Communications and Storage

There is currently no process for automated distribution of profile information outside PG&E.  An Internet website will need to be designed, tested, and documented to communicate profiles to ESPs, SCs, and other profile users.  Training will have to be provided for market participants to integrate the daily profile information into existing data systems designed for preparation of settlement ready data.  Modifications to PG&E systems for bundled service load PX bidding and PX/ISO settlement will also be required.

IV. THE IMPROVEMENT OF LOAD DATA ACCURACY

PG&E is committed to developing a data system by July 1st that provides the most accurate load profiling possible, in a reasonably timely fashion.  However, PG&E also respectfully submits that there are important unresolved issues with respect to dynamic load profiling that could pose impediments to improving load profile accuracy. In fact, the exact nature of some of these issues may not be fully evident until implementation is under way.

The crux of this issue is the policy tradeoff that must be established between data accuracy and timeliness of profile distribution.  While timeliness may be of value to market participants using profiles, for example in load forecasting applications, time is also required to ensure accuracy of the data.  As has been pointed out in this Report, traditional load research methods rely on VEE processes that use the load and billing data from the full revenue cycle and even the entire year to evaluate the validity of the data, perform editing and estimate values for load data points in question, and generate final class load estimates. It is clear that in a dynamic load profiling world, at least some of these methods will be impractical, by virtue of the fact that necessary data will not be available in the time frame allowed.

PG&E expects these and other dynamic profiling issues to be revisited during the course of workshops and proceedings in 1998.

Respectfully submitted,

LINDA L. AGERTER

PETER OUBORG

By____________________

PETER OUBORG

Law Department

PACIFIC GAS AND ELECTRIC COMPANY

P. O. Box 7442

San Francisco, CA  94120

Telephone:  (415) 973-2286

Fax: (415) 973-5520

Attorneys for

PACIFIC GAS AND ELECTRIC COMPANY

December 1, 1997

Table 1 Implementation Cost Estimates

|  | SAMPLE | FIELD WORK | DATA ACQ | VEE | ANALYSIS | STORAGE/ COMMUNICATION |
| --- | --- | --- | --- | --- | --- | --- |
| DYNAMIC PROPOSED for 7/1/98 | new TOU 10% over sample | 400 new 200 phone conversions 600 retrofits | -Weekly to Daily -Separate Process to retrieve DA/VDA from outside MDMA | -modify DA VEE for daily -adj for no current meter read(encoder too expensive) |  |  |
|  | Enhance tracking system | QA tracking | Upgrade & Enhance system | New system | New system | New system |
| Capital | $25,000 | $1,200,000 | $250,000 | $25,000 | $25,000 | $25,000 |
| Expense | $125,000 | $75,000 | $700,000 | $150,000 | $200,000 | $125,000 |
| Total | $150,000 | $1,280,000 | $950,000 | $175,000 | $225,000 | $150,000 |

Figure 1

“The UDCs agree that econometric techniques reflecting weather and other relevant variables would be an improvement [over static load profiling], but suggest that dynamic load profiling is preferable in the long term… The UDCs are not opposed to moving towards dynamic load profiles, although for PG&E and SDG&E, this is not feasible by 1/1/98,” Report on June 5, 1997 Direct Access Workshop on Load Profiling, p. 13.

Stratification is performed using sub-groups within each rate class to allow for a more efficient sample design.  For example, the residential class is stratified by usage and climate zone.

The current static load profiles for the residential and commercial time of use (TOU) customer segments are modifications of the larger class profile.  That is, the profiles are adjusted based on recorded sales by time of use period.  The shape within the TOU period remains the same as the overall class shape and is simply lowered or raised based on recorded sales for all TOU customers.  The dynamic load profile estimation process cannot be dependent on meter reads from the total TOU population because they will not be available each day, and therefore a separate sample will be required.

Such failures commonly result from, e.g. modem non-response, static phone line, disconnect prior to completion of download, etc.  Of those failures, 25% are estimated to be completed with additional attempts on the next day.  The remainder are added to the next night’s list of calls.  Persistent failure to connect results in a site being added to a problem report requiring a field inspection and possible replacement of the field equipment.

In some cases (e.g. where meters are remotely situated), phone line installation costs may be prohibitive and alternate sample points will need to be selected and installed.   An estimate of these sites is included in the estimated 400 new meters, described above.

Current requirements for non-UDC MDMAs in the Commission’s Proposed Opinion Regarding Meter and Data Communications Standards Workshop Report  (Metering PD) are based on a monthly meter read schedule for billing data retrieval.  Procedures developed for DA and VDA billing will require only a subset of customer meter reads to be retrieved on any given day, while all load research sample points will now require retrieval each day, in order to support dynamic load profiling.  Therefore, changes in the proposed interim Meter Reading and Meter Data Management Standards are required to accommodate daily access to load research sample meter data.

“Dynamic load profiles are created by reading the load research meters on a daily basis, and producing daily load shapes which reflect the actual usage for that customer segment for the day.”  (D. 97-10-086, p. 6)