Sierra Pacific Power Company California Load Profiles

Included is 3 workbooks (MS Excel) containing annual load profiles mapped to 1998.  These profiles are:  D1 for domestic class, A1/A2 for commercial class, and PA/OL/SL for agricultural/outside lighting/street lighting.  Each worksheet within the workbooks is a separate annual profile.  The columns represent time of day, in hourly intervals and the rows represent days of the year.  Each value in the spreadsheet represent the fraction of annual usage occurring in that interval.  A constant of 10,000 was used to allow resolution without a large number of significant digits.

Example of Load Profile Usage

- Add up the hourly values for the period in question.  For this example, residential class D1 on February 23 between 8:00am and noon, are used:

| Time of Day | Hourly Values from Worksheet |
| --- | --- |
| 8:00am | 1.28960 |
| 9:00am | 1.42450 |
| 10:00am | 1.41280 |
| 11:00am | 1.57990 |
| 12:00pm | 1.56170 |
|  | 7.26850 |

- Divide each interval value by the sum of all intervals.  These adjusted load profiles should add up to one for this period.

| Time of Day | Hourly Values from Worksheet | Sum of hourly values | Adjusted load profile |
| --- | --- | --- | --- |
| 8:00am | 1.28960 / | 7.26850= | 0.177423 |
| 9:00am | 1.42450 / | 7.26850= | 0.195983 |
| 10:00am | 1.41280 / | 7.26850= | 0.194373 |
| 11:00am | 1.57990 / | 7.26850= | 0.217363 |
| 12:00pm | 1.56170 / | 7.26850= | 0.214859 |
|  |  |  | 1.000000 |

3.  Multiply the consumption for the period by the adjusted load profile value.