SDG&E

Proposed Class Load Profiles and Associated Rate Schedules

Class Load Profile	Rate Schedule

Residential:  	DR, DR-LI, DM, DS, DT, DT-RV, D-SMF, EV-TOU, EV-TOU-2, EV-TOU-3, DR-TOU, DR-TOU-2

Small Commercial:	A, A-TC

Schedule AD:	AD

Medium Commercial/Industrial (<or=500 kW):	A-TOU, AY-TOU, AL-TOU, AL-TOU-C, AO-TOU, A-V1, A-V2, A-V3, A-V6-C, PA-T-1, RTP-1, RTP-2

Large Commercial/Industrial (>500 kW):	AL-TOU, AL-TOU-C, AO-TOU, A-V1, A-V2, A-V3, A-V6-C, NJ,

PA-T-1, RTP-1, RTP-2

Schedule A6-TOU:	A6-TOU

Agricultural: 	PA, PA-TOU

Lighting:	LS-1, LS-2, LS-3, OL-1, DWL

Use of SDG&E's Load Profiles

The SDG&E load profiles on this website are normalized to the 1998 calendar year; i.e., adding up all the hours for the year sum to 1.0.  To develop a load profile for a given consumption level for any given period of time, the following steps need to be performed:

1.	Add up the hourly values for the period in question.  For this example three hours will be used (e.g., Jan. 31 at midnight through Feb. 1 at 2:00 AM).

| Date | Hour |  | AL-TOU >500kW |
| --- | --- | --- | --- |
| 31-Jan-98 | 24 |  | 0.0000838663 |
| 1-Feb-98 | 1 |  | 0.0000813440 |
| 1-Feb-98 | 2 |  | 0.0000796591 |
|  | Sum of period: |  |  | 0.0002448694 |

2.	Divide the hourly values by the sum calculated in 1 above.  This newly created load profile will sum to 1.0 over the period in question.

| 0.0000838663 / 0.0002448694= 0.342493999 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0000813440 / 0.0002448694= 0.332193406 |  |  |  |  |  |  |
| 0.0000796591 / 0.0002448694= 0.325312595 |  |  |  |  |  |  |
|  |  | 1.000 |  |

3.	Convert the billing period’s consumption into an hourly load profile by multiplying the consumption by the newly created normalized load profile described in 2 above.

| Date | Hour | Normalized Load Profile | Total kWh in Period | Hourly Load |
| --- | --- | --- | --- | --- |
| 31-Jan-98 | 24 | 0.342493999 | 1,000 | 342.49 |
| 1-Feb-98 | 1 | 0.332193406 | 1,000 | 332.19 |
| 1-Feb-98 | 2 | 0.325312595 | 1,000 | 325.31 |
|  |  |  |  | 1,000.00 |