LOAD PROFILING WORKSHOP

OFFICE OF RATEPAYER ADVOCATES

JUNE 2, 1997

ALTERNATE PROPOSAL

ORA has emphasized the importance of distinguishing among load characteristics within customer classes as they are affected by pertinent factors like customer size, service on TOU rates, use of space heating, and daily temperature.  Some proposals have discussed the use of regression techniques to make such distinctions, and to make efficient use of the existing load research sample size.  The utilities have expressed reservations about modifying the structure of their load research programs by 1998.

A regression technique may satisfy all of these interests, by avoiding modifications of the existing load research programs and instead applying a formula (based on the existing programs) to customer billing data during the settlements process, to reflect the characteristics such as customer size.  Similar months could be grouped into seasons, and equations developed for each hour of the day, for each season, as predictors of customers’ loads.  Benefits of this approach could be assisting new ESPs with their load forecasting for bidding and scheduling, and simplifying the audit requirements that extensive segmentation of load profiles could produce.  Regression is a familiar technique to many participants in CPUC proceedings.

To illustrate this approach, ORA pooled the data used to produce Figures 6 to 9 of ORA’s main proposal into a single sample, treating the average summer weekday as a “day” with an assumed temperature of 85 degrees, and assuming a temperature of 105 degrees for August 30, 1995.  The dependent variable is hourly load in kW, while annual kWh is in kWh and “TOU” is 1 for TOU customers and “0” for non-TOU.  This example is intended only to illustrate the possible use of a regression technique, and this specific model formulation is not a recommendation by ORA at this time.  Additional discussion in technical workshops is recommended to develop appropriate models, if a regression technique is used.

Modeling of Residential Summer Loads

|  | Regression Output: |  | 7-8 AM | 5-6 PM |
| --- | --- | --- | --- | --- |
|  | R Squared |  | 0.5638 | 0.5665 |
|  | Degrees of Freedom |  | 1383 | 1383 |
|  | Constant |  | 0.103 | -0.681 |
|  | Regression Coefficient(s) | Annual kWh | 0.00011 (T=38.7) | 0.0002 (T=42.2) |
|  |  | Temperature | -0.0014 (T=0.8) | 0.007 (T=2.9) |
|  |  | TOU | 0.276 (T=4.2) | -0.803 (T=8.9) |