UDC / MDMA

Issues Template

Title:  		California Metering Exchange Protocols (CMEP) does not supply enough “flags”

to adequately  identify the different types of data being exchanged over the MDMA

server.

Background: 	At the January MDMA general meeting, each of the UDC’s identified a need for

flags and reached consensus.  It was decided to present this information to MDMA

users group. Issue was deferred to a later date.

Issue:		California Metering Exchange Protocols (CMEP) does not supply enough fields

to adequately identify the different types of data being exchanged over the MDMA

server

Alternative #1	Add   fields at the end of the record format for MEPMD01 and MEPMD02 to include

the following:

- C= Change meter, remove read;

- P= Change meter, set read;

- E= new service, set read (new service point)

- X= Remove meter (not replacing at this time)

- F= Final read (normal close of an account)

- S= Credit turn off read

- T= Turn on (initial read in system for opening an account)

- Z= Reset read (where read in system is manually set to a value)

Alternative #2	Allow MDMA users group to address the possibility of adding fields and

making recommendation to PSWG for implementation in 1999

Alternative #3	N/A

Recommendation: 	Alternative #1

Considerations:	Each of the UDCs have reached consensus on these codes.  Further discussion is

required for the appropriate field

Assigned to:	Larry Colwell  (SCE) / Ed McCann (SDG&E)

APPENDIX

Metering Service Data Records

(From PG&E Web Site - SDG&E CHANGES IN BOLD)

California Metering Exchange Protocol metering communications occur to

enable customer energy use and service billing. Most information

transmitted in Metering Service Data records consists of meter reading and

energy use data about specific customers.

Metering Service Data Records are typically generated by the MA and

supplied to the UDC and ESP. In the case of shut-off or turn-on, the UDC

may be responsible for generating appropriate MEPMD01 or MEPMD02 records with final and initial meter readings. In the case of a change out, the party changing

out the meter may be responsible for providing the final and initial meter

readings to the respective MDMA.  SDG&E has added a code at the end of the MEPMD01 and MEPMD02 records that will be available to flag a read as an opening or closing read.  As it is at the end of the record, it should not interfere with any party that doesn’t choose to look for it.

Metering Values vs. Date/Time Intervals

The values transmitted in Metering Service Data Records are typically an

accumulation of some quantity, such as kilowatt-hours, over an interval of

time. Each value has an associated Date/Time field as a time stamp. That

time stamp could conceivably identify the beginning or ending time of the

interval. Each of these approaches has advantages and disadvantages. The

California Metering Exchange Protocol uses end-of-interval Date/Time

time stamps.

Whether beginning or end of interval time stamps are used, a problem arises

that complicates totaling commodity usage on a day by day basis. Either the

first or last reading for a day will be listed with the previous or next

day's date. The use of end-of-interval time stamps does not complicate this

problem. Totaling algorithms must deal with the fact that the time stamp for

the last interval of the day will be for the beginning of the first

interval of the next day.

One potential solution to the end-of-interval totaling problem is to simply

offset the midnight reading to 23:59. This approach is inadequate simply

because billing days often do not begin and end at midnight. Some other

time of the day, such as 0700 local time is used. Offsetting all time stamps

by one minute might minimize this one minor totaling problem but would

introduce a systemic error in data representation. If such measures are

desirable, they should be applied to the data after it is transmitted via

California Metering Exchange Protocol.   SDG&E will provide interval time stamps using the midnight time of 24:00, but we will accept midnight times of 00:00 or 24:00.  MDMA’s should be cautioned that they include the 00:00 interval with the prior day even though most database systems would assign the following day’s date to that time.

Time-Of-Use Metering

There are two broad categories of metering data intervals employed for

different accounts. The first is pure interval values, such as those

accounts that are billed on total energy use on an hourly or monthly basis.

The second is what is traditionally known as Time-Of-Use where energy use

is broken into as many as five or more components. These components are

normally labeled "On-Peak", "Off-Peak", and so on. (SDG&E typically defines

three components: "On-Peak", "Semi-Peak", and "Off-Peak"). Time-Of-Use

billing has typically been applied to monthly totals of individual use

components but day-by-day billing is possible.

Time-Of-Use totaling is usually complicated by the treating of weekdays,

weekends and holidays differently when assigning usage to On-Peak,

Off-Peak, or other categories. Often, all day Sunday is treated as an

Off-Peak period. Time-Of-Use metering totals must therefore be identified

by their component names and the overall time interval for which they are

accumulated. Time-Of-Use and Interval data formats differ sufficiently to

justify supporting them with two separate record types. Interval data is

supported by "MEPMD01" - Metering Data Type 1 records and Time-Of-Use with

"MEPMD02" - Metering Data Type 2 records.  The three UDC’s have agreed to provide cumulative meter reads in the MEPMD01 format on a temporary basis.  By the time that our systems are ready to accept billing determinants using the MEPMD02 record, we will all convert to using the MEPMD02 record for cumulative reads as well.

Beginning and End of Month Meter Reads

Nominally, data transmissions of cumulative meter reads, such as monthly

meter reads, should include both beginning of period and end of period date

and usage values. This allows receiving entities to verify that no reading

overlaps or gaps have occurred.  SDG&E will only be providing end of the month meter reads

"MEPMD01" - Metering Data Type 1 - Interval Data

Interval Data is data that represents regular interval accumulation of

energy usage information, such as 15-minute, hourly, or daily, or monthly

accumulation or demand. Most energy metering information may be represented

using this record. The exception is traditional Time-Of-Use (TOU) usage

accumulation that has complex irregular interval definition. Monthly or TOU data may

be represented using "MEPMD02" record type.

"MEPMD01" represents a format to facilitate the transfer of metering data.

It is not intended to define how a utility customer's energy use is

administered or billed. "MEPMD01" supports single meter socket values.

Utility customers with more than one meter per account must be explicitly

handled. Billing for a single utility customer that involves the

aggregation of metering values may be done by some negotiated scheme by the

UDC acting as a metering agent. It could also be handled by using "MEPMD01"

records to transfer metering values separately, as separate metering

accounts, to be combined later in that customer's billing service. Both

methods are supportable by "MEPMD01".

The sequence of fields in this record is:

1. Record Type: Protocol Text: Always "MEPMD01"

2. Record Version: Date ("CCYYMMDD"): Currently "19970819"

3. Sender ID: Arbitrary Text: Identity of the entity sending this record.

It will typically be an abbreviation of the sender's company name.

Currently, only the first 16 characters of this field will be

recognized by PG&E.

4. Sender customer ID: Arbitrary Text: This is the senders identification

reference for the account to which this record applies. Currently,

only the first 12 characters of this field will be recognized by PG&E.

5. Receiver ID: Arbitrary Text: Identity of the intended recipient entity

of this record. It will typically be an abbreviation of the receiver's

company name. Currently, only the first 16 characters of this field

will be recognized by PG&E.

6. Receiver customer ID: Arbitrary Text: This is the receiving entities

identification reference for the account to which this record applies.

Currently, only the first 12 characters of this field will be

recognized by PG&E.

7. Time stamp: Date/Time ("CCYYMMDDHHMM"): Date and Time that this record

was created.

8. Meter ID: Arbitrary Text: This is the placard identifier or faceplate

serial number to physically identify a meter. For SDG&E, this is the number    assigned to the meter by the MDMA and reported to the Utility.  This is usually some

arbitrary combination of letters and numbers that make up a meter

manufacturer's serial number. It may, however, be some other easily

found identifying label on the metering equipment. This field may

optionally be used as a channel identifier for situations where that

information is useful. Currently, only the first 12 11 characters of this

entry will be recognized by PG&E SDG&E.

format 99999999CHN where CH is a literal and N represents the channel number for the meter and both are optional unless a channel scheme is required for metering.

9. Purpose: Protocol Text: Indicates the reason for this data

transmission. Defined values are:

o "OK" - Normal transmission.

o "RESEND" - Retransmission of previously sent data.

o "SUMMARY" - Summary of SP totaled data. Summary data usually

consists of values calculated from metering data such as monthly

totals calculated from 15 minute readings. This data is often

supplied on a regular basis (such as for quarterly reports).

o "HISTORY" - Archival account data. Archival data is retrieved

from long term storage and may be of lesser time resolution than

its original collection period. This data of generally supplied

once per request for analysis purposes.

o "PROFILE" - Account usage profile data.  N/A

o "TEMPLATE" - Account usage template data.

o “ADJUSTMENT” - Data is an adjustment for meter data previously sent.

10. Commodity: Protocol Text: Describes what commodity type this account

is for. Defined values are:

o "E" - Electricity.

o "G" - Gas.

o "W" - Water.

o "S" - Steam.

11. Units: Protocol Text: Describes the units of the data values. Examples

of values are: "KWHREG", "KWH", and "THERM". A complete list of

abbreviations is supplied in the Protocol Text Units listing. . Data

quality flags are used to indicate the raw, estimated, valid, etc.

status of values transmitted.

12. Calculation Constant: Numeric Floating-Point: Defines an optional

value which is used as a multiplier to convert data values to

engineering units. Typically this parameter is used with "PULSE" data

to allow calculation of equivalent "KWH" and "THERM" values. = 1.0

13. Interval: Time Interval ("MMDDHHMM"): Describes the time interval

between readings. Metering data is transmitted as Date/Time and value

pairs. In many cases, however, the time intervals for the data values

is so regular that Date/Time information past the first reading is

essentially redundant. This field may be used to minimize this

redundancy problem. If a Date/Time field, after the first reading in a

line, is empty, it is calculated by adding this interval to the

Date/Time of the previous value. This field is ignored if no empty

Date/Time fields are encountered in the record. This field is optional

if Date/Time fields are supplied for all values. = 00000015

14. Count: Numeric Integer: Indicates the number of Date/Time, flag, and

value sets to follow. A maximum of 48 sets is allowed per record.

= 32 (or 8 hours per record)

15. Data: Date/Time ("CCYYMMDDHHMM"), Protocol Text, and Numeric

Floating-Point triplet: Each data entry is a set of three fields. The

number of data entry sets is described in the "Count" field above.

format = 9.3

time is expressed in GMT

When an "Interval" field is supplied, Date/Time fields after the first

may left empty to be calculated when the record is read. An empty

Date/Time field is calculated by adding the time interval described in

the "Interval" field to the supplied or calculated Date/Time value of

the previous entry pair in this record. The Protocol Text field is an

optional field used as a data quality flag. Defined values are:

o (empty) - An empty flag field indicates that the value is OK and

validated.

o "E" - Value is estimated.

.

o "A" - Value is an adjustment. Adjustments are made to correct

metering inconsistencies or errors.

o "N" - Value is empty. No value is being sent for this interval.

May be sent as the first entry for a new account.

o "R" - Value is raw. No validation has been performed on value.

16.  Open/Closing Flag:  Protocol Text:  Flags record as being for opening or closing meter reads.  Current defined values are:

CODE		EVENT

o 	(empty) - An empty flag field indicates that the value is not an opening or 	closing read.

o	“C”-	Change meter, remove read

o	“P”-	Change meter, set read

o	“E”-	New service, set read (new service point)

o	“X”-	Remove meter (not replacing at this same time)

o	“F”-	Final read (normal close of an account)

o	“S”-	Credit turn off read

o	“T”-	Turn on (initial read for opening an account)

o	“Z”-	Reset read (where read in system is manually set to a value)

"MEPMD02" - Metering Data Type 2 - TOU Data

The sequence of fields in this record is:

1. Record Type: Protocol Text: Always "MEPMD02"

2. Record Version: Date ("CCYYMMDD"): Currently "19970819"

3. Sender ID: Arbitrary Text: Identity of the entity sending this record.

It will typically be an abbreviation of the sender's company name.

Currently, only the first 16 characters of this field will be

recognized by PG&E.

4. Sender customer ID: Arbitrary Text: This is the senders identification

reference for the account to which this record applies. Currently,

only the first 12 characters of this field will be recognized by PG&E.

5. Receiver ID: Arbitrary Text: Identity of the intended recipient entity

of this record. It will typically be an abbreviation of the receiver's

company name. Currently, only the first 16 characters of this field

will be recognized by PG&E.

6. Receiver customer ID: Arbitrary Text: This is the receiving entities

identification reference for the account to which this record applies.

Currently, only the first 12 characters of this field will be

recognized by PG&E.

7. Time stamp: Date/Time ("CCYYMMDDHHMM"): Date and Time that this record

was created.

8. Meter ID: Arbitrary Text: This is the placard identifier or faceplate

serial number to physically identify a meter. This is usually some

arbitrary combination of letters and numbers that make up a meter

manufacturer's serial number. It may, however, be some other easily

found identifying label on the metering equipment. This field may

optionally be used as a channel identifier for situations where that

information is useful. Currently, only the first 12 8 characters of this

entry will be recognized by PG&E SDG&E.

9. Purpose: Protocol Text: Indicates the reason for this data

transmission. Defined values are:

o "OK" - Normal transmission.

o "RESEND" - Retransmission of previously sent data.

o "SUMMARY" - Summary of SP totaled data. Summary data usually

consists of values calculated from metering data such as monthly

totals calculated from daily readings. This data is often

supplied on a regular basis (such as for quarterly reports).

o "HISTORY" - Archival account data. Archival data is retrieved

from long term storage and may be of lesser time resolution than

its original collection period. This data of generally supplied

once per request for analysis purposes.

o "PROFILE" - Account usage profile data. N/A

o "TEMPLATE" - Account usage template data.

o “ADJUSTMENT” - Data is an adjustment for meter data previously sent.

10. Commodity: Protocol Text: Describes what commodity type this account

is for. Defined values are:

o "E" - Electricity.

o "G" - Gas.

o "W" - Water.

o "S" - Steam.

11. Units: Protocol Text: Describes the units of the data values. Examples

of values are: "KWHREG", "KWH", and "THERM". A complete list of

abbreviations is supplied in the Protocol Text Units listing. Where

multiple unit types and seasons are transmitted, separate MEPMD02

records are sent for each. Data quality flags are used to indicate the

raw, estimated, valid, etc. status of values transmitted.

12. Season identifier: Protocol Text: This identifies the season for which

the values apply. Defined values are: "S" - Summer. "W" - Winter. This

field may be left blank for accounts that do not differentiate between

seasons. If this field is blank, it will be interpreted as indicating

winter for those accounts that do. A record may contain data for one

season only. Data for different seasons must be sent in separate records.

13. Calculation Constant: Numeric Floating-Point: Defines an optional

value which is used as a multiplier to convert data values to

engineering units. Typically this parameter is used with "PULSE" data

to allow calculation of equivalent "KWH" and "THERM" values.

14. Data Start Time: Date/Time ("CCYYMMDDHHMM"): Describes date and time

that the data interval reported in this record began.

15. Data Time stamp: Date/Time ("CCYYMMDDHHMM"): Describes date and time

that ends the interval reported in this record.

16. Count: Numeric Integer: Indicates the number of label-flag-value sets

to follow. A maximum of 6 sets is allowed per record.

17. Data: Protocol Text, Protocol Text, and Numeric Floating-Point

triplet: Each data entry is a set of three fields. A maximum of 6 sets

is allowed per record. Each set consists of a Protocol Text

Time-Of-Use component label field, a Protocol Text data quality flag,

and a Numeric Floating-Point value. The number of data entry sets is

described in the "Count" field above. Defined values for the quality

flag field are described in the "MEPMD01" record above. (An empty

indicates that the value is OK.) Defined values for the label field

are:

o "ON-PEAK"

o "OFF-PEAK"

o "PART-PEAK"

o "PEAK-2"

o "PEAK-3"

o "PEAK-4"

o "TOTAL"

o "SEMI-PEAK”

o "SUP-OFF-PEAK”

o "ON-PEAK-2”

o " SEMI-PEAK-2”

18.  Open/Closing Flag:  Protocol Text:  Flags record as being for opening or closing meter reads.  Current defined values are:

CODE		EVENT

o 	(empty) - An empty flag field indicates that the value is not an opening or 	closing read.

o	“C”-	Change meter, remove read

o	“P”-	Change meter, set read

o	“E”-	New service, set read (new service point)

o	“X”-	Remove meter (not replacing at this same time)

o	“F”-	Final read (normal close of an account)

o	“S”-	Credit turn off read

o	“T”-	Turn on (initial read for opening an account)

o	“Z”-	Reset read (where read in system is manually set to a value)

---------------------------------------------------------------------------