CEC STAFF PRESENTATION

LOAD PROFILING DATA AND INFORMATION ISSUES WORKSHOP

FEBRUARY 11, 1998

Goal:	Identify the type of information that the UDCs and other market participants should be required to maintain to assess the effects of load profiling and to determine how load profiling methodologies and administration can be improved.

I.	Power Exchange Price Variation

A.	Hourly PX prices need to be tracked and stored in a historical data base

1.	Provides a mechanism for review of the applicability of the PURPA precision criteria used in developing the UDCs’ class load profiles

2.	Develop monthly precision levels resulting from the current PURPA standard for each class load profile (average hourly or peak hour errors)

3.	Compare the PX price variation by month with that at time of system peak

4.	If a particular month’s price variation exceeds that of the system’s peak month then the class sample size should be increased to meet PURPA

5.	The class average load profile precision requirements resulting from tracking of PX prices should then be used as the required sampling criteria for testing the appropriateness of further segmentation

B.	Track Average PX Costs/Credits

1.	Develop average monthly PX costs/credits for the current load profile segments

2.	Estimate average monthly PX costs/credits for additional segments

--by climate zone for residential customers

--by building type for commercial customers

--other segmentation schemes, possibly ESP specific segments

3.	If the difference between the current class average PX cost/credit and a new segment exceeds some threshold, possibly ½ cent, then there may be a need for additional segmentation

4.	Estimate the total PX cost/credit differences between the class segment and each additional segment

II.	Update Dynamic Load Profiling Costs

PG&E, Edison, and SDG&E should systematically update the cost estimates of sampling and metering provided in the Dynamic Load Profiling Status Reports.  Such updates could include the availability of meters and reduced or increase costs resulting from restructuring.  The costs should be tracked in total and incrementally as changes in load profiling methodology and administration are needed.

III.	Develop Costs-Benefits Test

A.	Costs should include all the incremental expenditures needed to add additional segments or change methodologies and administration

--incremental sampling, metering, and communication costs

--incremental billing and software costs

--incremental processing costs

B.	Benefits should include the total reduction, by segment, in PX costs/credits imbalance resulting from the single class load profile

--benefits include the reduction of inequities (cross subsidization)

--systematic exclusion of certain customers from participation in direct

access due to the use of a single class average load profile

--alignment of actual energy costs to PX credits

IV.	Direct Access Participation

A.	Market Share of ESPs

--who are the customers being chosen for direct access

--is there a systematic bias

B.	Market Share of UDCs

--which customers have remained with the UDCs

--does the UDCs’ class samples still represent their customer base

V.	Information From The ISO On Unaccounted For Energy

A.	Is some entity going to keep track of UFEs, that is, maintain a historical database

B.	Will UFEs be separated between metered and load profile customers

VI.	Hourly Interval Meters—Availability and Costs

A.	Has direct access reduced the costs of hourly interval meters

B.	Are the meters available

C.	Are qualified meter installers available

D.	What are, if any, the log-jams in meter availability

E.	What level of consumption justifies the costs of hourly interval meters