Table 1:  Proposed Standards/Requirements for Meter Equipment in Direct Access

| Standards | Applicable for DA? | Required in CPUC’s D97-12-048 | Proposed Requirements | Recommended effective date | Comments |
| --- | --- | --- | --- | --- | --- |
| ANSI C12.1-1995, Code for Electricity Metering | Yes | Yes | Yes | Now | To be used in conjunction with Appendix B of Bob Lane letter |
| ANSI C12.4 Mechanical Demand Registers | No | No | No | None |  |
| ANSI C12.5, Thermal Demand Meters | No | No | No | None |  |
| ANSI C12.6 Marking & Arrangement of Terminals for Phase Shifting Devices used in Metering | Yes | Yes | No | Now |  |
| ANSI C12.7 Watt-hour Meter Socket | Yes | Yes | Conditional yes | Now | Applies only to socket meters |
| ANSI C12.10 Electromechanical Watt-hour Meters | Yes | Yes | Conditional yes | Now | Applies only to Watt-hr meters |
| ANSI C12.11, Instrument Transformers for Revenue Metering, 10 kV-350 kV BIL (0.6-69 kV NSV) | Yes | Yes | Yes | Now |  |
| ANSI C12.13, Electronic TOU Registers for Electricity Meters | Yes | Yes | No | Now |  |
| ANSI C12.18, Type 2 Optical Port | Yes | No | Conditional yes | Now | Applies if port is provided |
| ANSI C12.19, Utility Industry End Device Data Table | Yes | No | Conditional yes | 2 years from publication date | Applies when data retrieved electronically (not visually) |
| ANSI C12.20, 0.2% & 0.5% Accuracy Class Meters (approved but not yet released) | Yes | Yes | Conditional yes | Conditional yes | Applies if meters are solid state |
| ANSI C12.21, Protocol Specification for Telephone Modem (not approved) | Yes | No | No | 2 years from publication date |  |
| ANSI C12.22, Meter Interface to Network Protocol Gateway (not approved) | Yes | No | No | 2 years from publication date |  |
| ANSI C37.90.1, Surge Withstand Test | Yes | Yes | Yes | Now | Adds to ANSI C12.1 |
| ANSI 57.13, Instrument Transformers (70 kV - 230 kV NSV) | Yes | No | Yes | Now |  |
| ANSI Z1.4 and ANSI Z1.9, Sampling Procedures and Tables for Inspection | Yes | Yes | Yes | Now |  |
| FCC Regulations, Part 15, Class B | Yes | Yes | Yes | Now |  |
| MTR1-96 Specification for ISO Applications (not required for all meters used in DA) | Yes | Yes | Conditional Yes | Now | Applies only certification test section of MTR1-96 |
| NIST Standards (specific standards to be clarified) | Yes | Yes | Yes | Now |  |

Additionally, the following alternative meter equipment certification testing and enforcement process is suggested for the group’s discussions and considerations:

Certification testing:

- Use ISO certification and 4 additional tests to cover communication testing and sun-light affect testing, or

- Use CPUC’s decision D97-12-048, or

- Use Appendix B of Bob Lane’s document.

Enforcement Process:

- Manufacturer issues a certification testing document which is verified and approved by an ESP(MSP)/UDC (user of meters), and

- Manufacturer sends a copy of its certification testing document to the CPUC for access to public review or posting on a web site.