REQUIREMENTS FOR

MEASUREMENT AND DATA COMMUNICATION EQUIPMENT

Submitted to PSWG Subgroups 1,2, and 3

Michael R. Jaske

California Energy Commission

March 6, 1998

Prior to selecting prescriptive equipment standards or system performance standards, it is essential to understand the business and operational requirements such standards are intended to support.  These include the requirements for data measurement, data communication frequency, measurement device accessibility, redundancy of capabilities to reduce impacts of equipment failure, and other capabilities.  This brief paper enumerates a series of  measurement and data access requirements that appear to have been established by prior CPUC and/or ISO decisions.  It also raises a series of  desirable performance capabilities, and usage measurement reliability and verification issues that may also be requirements, but which are less clearly mandated by prior CPUC or ISO decisions.  Prior to in-depth discussion of equipment and data communication standards, these requirements should be reviewed and any outstanding issues resolved.  The PSWG must, at a minimum, address the mandated requirements.  Beyond these, the PSWG must decide which (if any) additional business requirements it wants to support by evaluating relevant standards.

In addition, it is important to recognized that requirements vary quite considerably from one customer to another, sometimes as a result of CPUC-approved rate designs and sometimes from differential requirements of participants in one or more of the new markets created by restructuring.  Therefore, it is important to determine how various measurement requirements, meter access requirements, and data communication frequency requirements result in subgroups of customers with homogeneous requirements.  It may be necessary to evaluate the applicability of various standards to each of these subgroups individually, rather than to assert that a given standard is or is not applicable to California as a whole.

1. Measurement and Data Access Requirements

Measurement requirements stem from a series of CPUC decisions, most of which have been made in general rate cases concerning rate design and billing determinants.  Under the rate freeze imposed by AB 1890, these billing determinants must continue to be used.  In a few limited instances, the ISO’s requirements supplement those of the CPUC.  Where a customer has both CPUC and ISO requirements, the most stringent of the two should be used to define a subgroup, and to form the basis for reviewing relevant standards.

Data access requirements have changed radically as a result of  industry restructuring.  Rather than the once near universal collection through manual meter reading on an approximately monthly interval (whether cumulative energy, TOU time period energy, or hourly energy recordings), the structure of the Day Ahead energy market, the Hour Ahead energy market, and the ancillary services markets place great emphasis on more rapid acquisition of data in addition to the clear emphasis on measurement of data on short time intervals (hourly for most purposes).  An issue that must be resolved is whether a metering and data communication system “requirement” exists in those instances where it is to the advantage of the customer or its energy service provider to have rapid access to measurements, but where this is not an absolute “requirement” in the sense of CPUC tariff billing determinants.

a. Billing Determinants for SCE Tariffs

The following items were identified in a quick review of SCE’s tariffs.  Other utilities have comparable measurements that their tariffs require for some customers.  There may be a few additional measurements that are also needed for limited sets of customers.

(1) cumulative kWh

(2) TOU time period kWh

(3) maximum kW in any 15 minute period

(4) maximum kW in any 15 minute period within each TOU time period

(5) highest kVA in any 15 minute period

(6) excess transformer capacity measured in kVA

(7) 3-phase vs. single phase

(8) horsepower rating of a motor

(9) type of luminaire

b. Intervals of Measurement

Traditional reliance upon cumulative energy over the entire billing period or cumulative energy within broad TOU time periods is being supplemented for large numbers of customers by the ISO and PX market requirements to use hourly energy as the basis for load forecasting/bidding/scheduling and energy imbalance settlements.

(1) cumulative kWh for the billing period for most residential and small commercial customers receiving bundled service, and those direct access customers eligible for load profiles

(2) cumulative kWh by TOU period for residential and small commercial customers on TOU rates

(3) kWh by hour for all direct access customers not on load profiles

(4) kWh by 10 minute interval for those direct access participants who elect to offer some or all of their load to the ISO as an ancillary service, and who are required to have 10-minute interval measurement capabilities to ensure that their offers, if accepted, are provided.

c. Access to the Measurement Device

Known Requirements for Access to Data.  Except for a small set of customers, their are no requirements for access to the measurement device or an ancillary storage device.  Virtually all customer’s billing determinant needs can be satisfied by data obtained within 5 days of the close of meter reading period.  The following requirements are known:

(1) standard: data about usage obtained within 5 days of the end of a meter reading period

(2) direct access load forecasting sample: data about usage obtained daily

(3) ISO grid-connected customers: the meter must be directly pollable by the ISO

There were no “requirements” that measurement devices be remotely accessible until the electricity restructuring process resulted in the formation of the ISO.  The ISO requires that large customers directly connected to the ISO grid have meters and metering telemetry that supports its MDAS process.  Through this system, the ISO can directly poll the meter to determine usage.

Commercial Requirements.  There are business advantages to customers and their suppliers to have access to usage measurements on a rapid basis, but these are not “requirements” in the mandated obligation; rather, they are “requirements” in the sense of the desirable specifications of a commercial, business system.  For some customers, rapid access to usage measurements means directly pollable meters to permit close matching of actual loads with ISO scheduled loads.  The Hour Ahead energy market operated by the ISO motivates some customers and their suppliers to have these capabilities.  Other customers and their suppliers will be satisfied with daily uploading of hourly usage measurements from the previous day.  These data would be used to make load bids to the PX or to submit load schedules to the ISO in their respective Day Ahead energy markets.

2. Usage Measurement Reliability and Verification

The CPUC has traditionally valued usage measurement reliability and verification.  It has required various standards of accuracy for meters themselves.  It has required utilities to install a visually readable display for customer verification, when electronic meters and telemetry would have sufficed for billing purposes.  In its various direct access and revenue cycle unbundling decisions, the CPUC has only implicitly touched on these topics, but it can be presumed that what it has traditionally valued, it continues to value in the new paradigm of competitive services.

a. Measurement Accuracy

Traditional electromechanical meters measuring cumulative kWh achieved a level of consistent accuracy over very long lifetimes believed to be consistent with the needs of the industry.  Given the new structure of the industry, the competitive choices for energy and revenue cycle services available to customers, and the increased pace of technological change, rethinking measurement accuracy may be important.

Issues to be Resolved

i. Given the shifts toward hourly energy as the fundamental unit of commerce for energy, how should accuracy for kWh be defined?  Should accuracy be defined in over a short time interval, rather than over a long time interval?  Should energy accuracy be defined in a way that is independent of local or system frequency departures from 60 Hz?

ii. What standards should be required for the other measurements used in larger commercial and industrial tariffs?

iii. Are electronic meters capable of a different measurement accuracy standard?  If they are, should this capability be used?

iv. Is there a need to establish a long term measurement accuracy requirement to ensure that meters perform acceptably over their lifetimes?

b. Field Readable Capability

Some SCE tariffs permit the customer to request that the utility install a field-readable display so that the customer can visually read the meter.  This feature can also provide some benefits as a backup if the telemetry system normally used fails.

Issues to be Resolved

i. Should a field-readable display be required?  If so, should this requirement be satisfied with a prescriptive equipment standard or a performance standard?

c. Usage Measurement Data Storage

In D.97-10-087, the CPUC requires an MDMA to store usage data for three years as an aid to resolving billing disputes.  This requirement presumes that data has been uploaded from the measurement device in the field.  Some problems with measurement and/or meter reading could be reduced by a requirement to have the meter or an associated device on the customer’s premise store measurements for later retrieval when the principal meter reading process fails.

Issues to be Resolved

i. Should a customer premise storage device be required to temporarily store meter measurements to increase the reliable measurement and communication of actual customer usage patterns for billing purposes?