Revised Strawman Recommended California Standards Connecting an

Electric Meter to a Telephone Network via an RF Data Concentrator

An Initial Contribution by

Bill Rush and Vic Tamosaitis

Institute of Gas Technology

Version of April 8,1998

Topology

This document is intended to stimulate discussions by describing a specific implementation of a remote electrical meter reading system.  The general topology, taken from IEEE 1397, is shown in Figure 1.  The IEEE 1397 topology, as adapted for California electric meters, is shown in Figure 2.  The topology for linking electric meters, via an RF based data concentrator, to the telephone network is shown in Figure 3, as a subset of the general California model.  Interfaces and devices addressed within the scope of this document are indicated by white blocks and solid lines.  Null devices are cross-hatched.  Devices and interfaces not addressed are shaded gray.  Functionally, this implementation is a meter connected via an RF link to a data concentrator, which is then interfaced to the telephone Wide Area Network (WAN).

Reference Model

The system described is based upon the standard reference model, also set forth in IEEE 1397.  The reference model described in IEEE 1397 is shown connected to an RF data concentrator/router in Figure 4.  The specific RF data concentrator to telephone implementation addresses the following- all of the layers within the concentrator/router, the physical layer interface up through the analysis layer within the meter, and the primary element and signal conditioning layers associated with the meter.  The two layers directly above the application service - storage and digital conversion - are typically enclosed within the meter, so they are left to the manufacturer’s discretion.

Recommended Standards for Interface

Specific assignment of standards for the services (layers) being provided shall be as follows-

Meter

| Physical layer: |  |
| --- | --- |
| Data Link layer: | ISO 7776 ADLC (Version of High-level Data Link Control Procedures being progressed through ANSI LLC 802.2 ISO 8802-3 LAN Carrier Sense Multiple Access with Collision Detection |
| Network layer: | X.233 CLNP |
| Transport layer: | X.224 TP X.237 CLTP |
| Session layer: | X.225 Session Protocol |
| Presentation layer: | X.226 Presentation Protocol |
| Application layer: | ISO 9506-1, 2 Manufacturing Message Specification- Part 1: Service Definition; Part 2: Protocol Specification Incorporate PSEM packets in MMS envelope. |
| Analysis service: | ANSI C12.19 = IEEE1377 = Utility Industry End Device Data Tables = “Tucker Tables”- Use most recent version since document is under revision. |
| Primary Element and Signal Conditioning: | KYZ Standard Pulser Interface |

Router

RF SIDE

| Physical layer: |  |
| --- | --- |
| Data Link layer: | ISO 7776 ADLC (Version of High-level Data Link Control Procedures being progressed through ANSI LLC 802.2 ISO 8802-3 LAN Carrier Sense Multiple Access with Collision Detection |
| Network layer: | X.233 CLNP |
| Transport layer: | X.224 TP X.237 CLTP |
| Session layer: | X.225 Session Protocol |
| Presentation layer: | X.226 Presentation Protocol |
| Application layer: | ISO 9506-1, 2 Manufacturing Message Specification- Part 1: Service Definition; Part 2: Protocol Specification Incorporate PSEM packets in MMS envelope. |
| Analysis service: |  |

Telephone side

| Physical and Data Link layers: | IEEE 1390, Standard For Automatic Meter Reading via Telephone IEEE P1390.1, Standard For Automatic Meter Reading via Telephone – Profile Protocols for Switched Telephone Network IEEE 1390.2, Standard For Automatic Meter Reading via Telephone – Network to Telemetry Interface Unit |
| --- | --- |
| Network layer: | RFC 791 Internet Protocol, IP v4 |
| Transport layer: | RFC 793 Transmission Control Protocol |
| Application Layer: | IS0 9506-1,2 Manufacturing Message Specification-Part1: Service Definition; Part 2: Protocol Specification Incorporate PSEM packets in MMS envelope. |
| Analysis Service: |  |

Figure 1 – IEEE 1397 Topology

Figure 2 – IEEE Topology Adapted for California Electric Meters

Figure 3 – IEEE1397 Implementation Connecting Electric Meters to Telephone via an RF Data Concentrator

# Figure 4 – IEEE 1397 End Device Reference Model and Data Concentrator