Interpolation when there is a Power Outage

The following table shows some of the possible ways to interpolate missing intervals when they are adjacent to a power outage.    Before the latest revision of the VEE rules, the interpolation was performed as in Example A.

In the most recent revision of the rules, we added the statement "Intervals containing a power failure cannot be used as end points for interpolation".    I have a question about how to implement this rule.    Kathy responded, “My understanding is that an interval with a power failure would be treated the same way as an interval that isn't valid.    You would find the closest valid interval that is not a power failure to use as the end point.”

Kathy’s response leaves some ambiguity.    If I treat a power failure “as an interval that isn’t valid”, I would estimate it (Example B).    But that is inconsistent with our treatment of power failures in the remainder of our data.    If I choose the “closest” valid interval that is not a power failure as the end point, does that mean the closest valid interval before the power failure or closest to the data we are estimating?    In the example given, the closest valid interval to the data needing estimation is our end point.    Should I use that interval is my first and second point for interpolation (Example E)?    If we specifically mean the closest valid interval before the power outage, the first interpolation point is the interval at 4:45.    If we use this as the first point for interpolation, and we are not interpolating across the power outage, should we fill the missing intervals as though the first interpolation point occurred at 5:30 (Example D)?    Or should we interpolate using the actual elapsed time but fill only the missing intervals and not the power outage intervals (Example C)?

| Time | Raw Data | Status | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4:30 | 111.71 | Good | 111.71 | 111.71 | 111.71 | 111.71 | 111.71 |
| 4:45 | 112.34 | Good | 112.34 | 112.34 | 112.34 | 112.34 | 112.34 |
| 5:00 | 140.70 | Power Out | 140.70 | 134.25 | 140.70 | 140.70 | 140.70 |
| 5:15 | 0.00 | Power Out | 0.00 | 156.17 | 0.00 | 0.00 | 0.00 |
| 5:30 | 0.00 | Power Out | 0.00 | 178.08 | 0.00 | 0.00 | 0.00 |
| 5:45 | . | Needs Estimate | 81.27 | 200.00 | 200.00 | 156.17 | 243.83 |
| 6:00 | . | Needs Estimate | 162.53 | 221.91 | 221.91 | 200.00 | 243.83 |
| 6:15 | 243.83 | Good | 243.83 | 243.83 | 243.83 | 243.83 | 243.83 |
| 6:30 | 245.55 | Good | 245.55 | 245.55 | 245.55 | 245.55 | 245.55 |
| 6:45 | 249.67 | Good | 249.67 | 249.67 | 249.67 | 249.67 | 249.67 |

Note:    The values used as end-points are shown in Bold and Underlined.    The estimated values are shown in Italics.

###### Example A

Treat the power outage as though it is valid data.    Interpolate using the power outage as an end-point.

Advantage:    If the data collected during the power outage represents actual usage, this seems like the most accurate approach.

###### Example B

Treat the power outage as though it is data that needs estimating.    Interpolate using the last "good" data point as the first point for interpolation.

Disadvantages:

- We do not typically treat power outages as bad data.

- If the power outage is two-hours or more, it is now unclear whether we should be using interpolation or the average profile to estimate the data.

###### Example C

Use the closest valid interval before the power outage as the end point.    Interpolate using the actual elapsed time.    Do not estimate the intervals with the power outage.

Disadvantage:    Depending on the length of the power outage, the last good point before the power outage may not be representative of usage during the period needing estimation.

###### Example D

Use the closest valid interval before the power outage as the end point.    Interpolate as though no time elapsed during the power outage.    Do not estimate the intervals with the power outage.

Disadvantage:    Depending on the length of the power outage, the last good point before the power outage may not be representative of usage during the period needing estimation.

- Example E

Use the one good interval that is adjacent to the period needing estimation as the first and second point for interpolation.    Do not estimate the intervals with the power outage.

Disadvantage:    We need an additional rule for dealing with the case when there is a power outage on both sides of the period needing estimation.