# Introduction

The VEE subcommittee held a series of conference calls and meetings to review the Interval VEE Rules 1.3 and develop VEE rules for monthly data.  Notices for all meetings and conference calls were distributed to the PSWG email exploder and website, as well as anyone who asked to be put on the mailing list; they were also announced at the joint UDC/MDMA meetings.  The meetings and calls were open to all interested parties.  Notes for each meeting or call were also distributed to the PSWG email exploder and website, as well as interested parties.

Several principles developed during the course of the meetings, which apply both to the work this spring as well as to any future work:

- The rules should promote fairness in the marketplace

- The goal of the rules is to provide quality data

- Solutions must fit the magnitude of the problem - when evaluating solutions, the costs must be considered against the frequency of occurrence and the quality of the data

- Modifications to the rules should typically be required when they result in a significant improvement in the data quality

- When modifications to the rules are made, reasonable implementation plans should be defined allowing time for all parties to comply

- Variations for different technologies should be allowed where appropriate

# Interval Data

Six conference calls were held to review the Interval VEE Rules 1.3 published in December, 1997, culminating in a final review meeting on June 18, 1998.  Participation in the meetings was as follows, with a Y indicating at least partial attendance:

| Company | 4/7 | 4/20 | 5/8 | 5/18 | 5/27 | 6/5 | 6/18 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ABB | Y | Y | Y | Y | Y |  |  |
| Cellnet | Y | Y | Y | Y | Y |  |  |
| Energy Interactive | Y | Y |  | Y | Y |  |  |
| Energy Management Services | Y |  | Y |  |  |  |  |
| Enron |  |  |  | Y |  |  |  |
| eT | Y | Y |  |  |  |  |  |
| FirstPoint |  |  | Y |  |  |  |  |
| ORA | Y | Y |  |  |  |  |  |
| PG&E | Y | Y | Y | Y | Y |  |  |
| SCE | Y | Y | Y |  |  |  |  |
| Schlumberger |  |  |  |  | Y |  |  |
| SDG&E | Y | Y |  |  |  |  |  |
| Sierra Pacific | Y | Y |  |  |  |  |  |
| Stardata |  |  |  |  |  |  |  |

The group reviewed the existing rules in light of market startup, clarified the rules, and addressed situations that were not addressed previously due to time constraints.  The detailed rules are included in Appendix XXXX.  Major changes include the following:

- Spike check threshold - For very low usage customers, a valid pulse count of a few pulses may result in failing the spike check.  An optional minimum threshold was added to allow MDMAs to automate checking for this condition and passing the data.

- kVARh check threshold - For very low usage customers, a valid pulse count of a few pulses may result in failing the kVARh check.  An optional minimum threshold was added to allow MDMAs to automate checking for this condition and passing the data.

- Use of partial days as reference data for estimation - Days containing less than 24 hours of good interval data may be used as reference data to estimate data for other days

- Use of days containing power failure as reference data for estimation - Days in which a power failure occurred should not be used as reference data for estimation.

- Use of accurate meter readings to scale estimated intervals - When data is estimated based on historical data, and accurate meter readings or usage are available, the estimated data can be scaled based on the actual usage.

- Simplified proration algorithm (when meter clock is off) - A simpler method to prorate data when the meter clock was off is provided.

- Automating handling of irregular usage customers - Rules are provided to determine which customers are irregular usage, and how special tests can be designed and automated for those customers.

- Test mode intervals - Interval data collected when the meter was in test mode should not be used as actual data; they should be estimated.

Only two of the above changes were considered required by the group; the rest are optional and may be implemented at the MDMA’s discretion.  The proposed implementation plan for the changes is described in the following table.  The Optional/Required column indicates if market participants will be required to make this change.  The Earlist date acceptable column indicates the earliest a market participant is allowed to implement this change (note to UDCs - this means the VEE test would need to allow these options), and the Required by column indicates the date by which market participants must implement the option (only applies to required options).  During the discussion, it was noted that some of the optional changes have a bigger impact on some technologies than others.

| Modification | Optional/Required? | Earliest date acceptable | Required by |
| --- | --- | --- | --- |
| Spike check threshold | Optional | Now | n/a |
| kVARh check threshold | Optional | Now | n/a |
| Use of partial days for estimation | Optional (may make a bigger difference with some technologies than others) | Now | n/a |
| Don’t use days containing power fail as source for estimation | Required | Now | 1/1/99 |
| Allow use of accurate meter readings scale estimated data | Optional (may apply more to some technologies than others) | Now | n/a |
| Simplified proration algorithm when meter clock is off | Optional | Now | n/a |
| Automated handling of irregular usage | Optional | Now | n/a |
| Handling of test mode intervals | Required | Now | 1/1/99 |

The group felt examples and flowcharts would be helpful to new market participants, but did not have time to develop them.  Also, the checks should be reviewed for effectiveness after there is more actual market experience; checks that don’t uncover expected problems should be modified or deleted, and checks may need to be added to uncover problems not foreseen when the tests were written.

# Monthly Data

Four meetings were held to develop rules for monthly VEE, based on Decision  97-12-048 dated December 3, 1997.  Participation in the meetings was as follows:

| Company | 4/27 | 5/6 | 5/27 | 6/10 | 6/17 |
| --- | --- | --- | --- | --- | --- |
| ABB | Y | Y | Y |  |  |
| Cellnet | Y | Y | Y |  |  |
| Energy Interactive | ? |  |  |  |  |
| Energy Management Services | ? |  |  |  |  |
| Enron | ? | Y | Y |  |  |
| eT | ? |  | Y |  |  |
| FirstPoint | ? | Y |  |  |  |
| ORA | ? |  |  |  |  |
| PG&E | Y | Y | Y |  |  |
| SCE | Y |  |  |  |  |
| Schlumberger | ? |  |  |  |  |
| SDG&E | Y | Y | Y |  |  |
| Sierra Pacific | ? |  |  |  |  |
| Stardata | ? | Y |  |  |  |

The group further defined the monthly validation and estimation rules found in the Commission ruling, as well as defining estimation rules for TOU data.

# Recommendations