## Simplified Pro-rating Algorithm

The objective of this algorithm is to create the expected number of intervals while preserving the usage recorded in the actual number of intervals.

### Case 1: Actual number > Expected Number

In the case where the actual number of intervals is greater than the expected number of intervals, truncate the extra intervals.  The total usage will decrease by the amount recorded in the truncated intervals.  In order to preserve the recorded usage, the usage in the remaining intervals will need to be scaled up by an adjustment factor as described below.

### Case 2: Actual number < Expected Number

In the case where the actual number of intervals is less than the expected number of intervals, interpolate using the last good interval to create the needed intervals.  The total usage will increase by the amount in the interpolated intervals.  In order to preserve the recorded usage, the usage in each interval will need to be scaled down by an adjustment factor as described below.

The adjustment factor is the ratio between the recorded usage and the pre-adjusted usage, as shown below.

Recorded Usage = Total usage for the period.  It can be derived from the sum of the usage in the “actual” intervals or from good meter readings.

Pre-adjusted Usage = The total usage in the intervals after they have been truncated or interpolated to the expected number of intervals.

Adjustment Factor = (Recorded Usage/Pre-adjusted usage)

Note that the Adjustment Factor will be greater than one in Case 1 and it will be less than one in Case 2.  Multiply the usage in each interval by the adjustment factor to create adjusted usage.  The sum of the usage in the adjusted intervals should now be equal to the recorded usage.  Flag all intervals for the period as estimated.