Notes from INTERVAL VEE Conference Call 5/8/98

Participants:

Kevin Simonson - Energy Management Services

Jayne Collins, Valerie Nibbler, Duncan Cano - PG&E

Carmina Luna - SCE

Tom Lofgren - Cellnet

Amos Tsikayi - FirstPoint

Kathy Smith - ABB

Review Interval Data Action Items, Draft 4

- Amos Tsikayi of FirstPoint presented a proposal for an alternate estimation algorithm for the case where the data needing estimation is >2 hours long.  The alternate is used when you have known good meter readings (or total usage) for the time period, and uses the actual usage to scale the values obtained using the existing algorithm.  The group agreed the proposal would provide more accurate data and could be used providing the MDMA knows the meter readings/total usage are accurate.  It would be identified as a separate estimation algorithm.  It will be added into the text of the next revision of the VEE rules.

- The group agreed that partial days (either incomplete days due to meter installation or days with estimated data) can be used as source data for estimation provided the day did not include any power failures and the specific intervals used as source data are not estimated.

- The group agreed that days with power outages should NOT be used as source data for estimation, as the day does not represent typical usage.  No-one in the group was familiar with how recorders report power outages and how they could be differentiated from the situation where the recorder received no pulses from the meter; participants who use recorders agreed to investigate and report back.

- The wording “similar day” in section 4.2.4 has been confusing and will be changed to “same weekday”.

- Section 3.4.4.2 will be changed to reflect <= 4 pulses, not < 4 pulses.

- The group agreed that <=10 pulses is valid in 3.4.3.1.

- The group agreed to leave “like days” in for estimation of a > 2 hour gap.

- The group started discussing how to manage irregular usage customers.  An irregular usage customer is one whose normal load pattern will cause them to fail a validation test every month.  The group agreed that it would be good to automate handling these exceptions as much as possible.  A time period of 3 months verification of actual usage was proposed before a customer could officially be labeled as an irregular usage customer.  The group is to consider how to best manage these sites - possibilities might be to exempt them from the specific validation test, using a different range/tolerance on the validation test, etc.  This will be discussed at the next meeting.

- It was brought to the group’s attention that the MV-90 does not export the data required by the algorithm in Appendix A.  The data is available for manual inspection.  Since this condition is very rare, alternate proposals should be considered for estimation techniques when the meter time has drifted.  Participants should submit alternate proposals for consideration by the group.  One alternative mentioned was when there are too few intervals, pad the data out with zeros and then use the appropriate estimation algorithm (<2 hours or > 2 hours); if there were too many intervals, truncate them (if you expected x intervals, use the first x intervals of the data retrieved and throw the rest away) and mark them as estimated.

Next Steps:

There will be a conference call Monday, May 18, at 10:00 am PDT (1:00 pm EDT).  The dial in number is 888-232-0365, and the Participant Code is 253320..  Areas to be discussed include:

- Review power failures for recorders

- Review Appendix A and alternate proposals

- Discuss irregular usage customers

- Discuss how to handle intervals flagged “test mode”

- Discuss list of holidays to be used for estimation

- Review changes to draft document

- Discuss recommended implementation schedules for changes

- Review Appendix D

New INTERVAL Action Items:

- Participants who use recorders to investigate how they report power failures.

- Participants to submit alternate proposals for Appendix A.

- Participants to consider how to best manage irregular usage customers.

- Kathy Smith to update document.

Action Items from 5/6 MONTHLY Meeting:

- UDCs to provide what they presently do to estimate TOU data

- Kathy Smith to contact Steve Roscoe re: do load profiles contain average usage data and will there be a load profile for every TOU rate

- Kathy Smith to circulate a request to UDCs and MDMAs for areas where the 1.3 rules were unclear or led to confusion.

Action Items from 4/27 MONTHLY Meeting:

- PG&E to circulate information regarding monthly hi/lo usage check

- SDG&E to prepare data from hi/lo demand check based on data collected so far

- PG&E to prepare alternate proposal for demand reasonableness check

- Kathy to revise document based on meeting

Action Items from 4/20 INTERVAL conference call:

- UDCs to review spike check thresholds of 10 pulses - CLOSED.  <= 10 pulses is ok)

- UDCs to prepare to present reasons why a whole day’s data should be considered suspect and not used for estimation if one interval of that day is estimated - CLOSED - partial days may be used subject to rules listed in document

- UDCs with rate schedules with different “like days” (i.e., MWF are like days, not MTWThF) are to consider the affect of sections 4.3.4 through 4.3.6 - CLOSED - like days will stay as they are

- Kathy Smith to revise VEE rules based on discussions - CLOSED - draft 4 was distributed

- Kathy Smith to send nationalization proposal to PSWG for vote - CLOSED - submitted to MDMA subgroup

- Kathy Smith to check with Bill Rush about eventually moving VEE to IEEE SCC 31 - OPEN

- Carmina Sara to check for additional purses for the kVARh check - OPEN

- Kathy Smith to check with Ed McCann about presentation at UDC/MDMA meeting - CLOSED.  VEE update was presented.

- Kathy Smith to check with SDG&E about having VEE meeting after UDC/MDMA meeting 5/5. - CLOSED.  Meeting was held for monthly data.

Action Items from 4/7/98 INTERVAL Conference Call:

- John VanderLinde to provide description of the proposed demand tolerance check p - CLOSED.  The proposal to add the additional check has been withdrawn.

- All participants to consider should VEE rules be moved toward national standardization - CLOSED.

- UDCs to provide thresholds for spike and kVARh checks.  CLOSED

- Tom Lofgren to draft proposal to resolve timing of MDMA receipt of customer historical data for joint UDC/MDMA meeting.  OPEN

- Kathy Smith (with help from UDCs) to draft proposal to discuss adjustments for joint UDC/MDMA meeting.  CLOSED - draft submitted to SDG&E

- UDCs to prepare to present reasons why a whole day’s data should be considered suspect and not used for estimation if one interval of that day is estimated.  CLOSED

- UDCs with rate schedules with different “like days” (i.e., MWF are like days, not MTWThF) are to consider the affect of sections 4.3.4 through 4.3.6.  CLOSED

- Kathy Smith to revise VEE rules based on discussions.  CLOSED - Draft 2 of version 1.4 was published.