Recommended California Standards Connecting an

Electric Meter to a Telephone Network

An Initial Contribution by

Bill Rush and Vic Tamosaitis

Institute of Gas Technology

Version of March 25, 1998

Topology

This document describes a specific implementation of a remote electrical meter reading system.  The general model model, taken from IEEE1397, is shown in Figure 1.  The IEEE 1397 topology, as adapted for California electric meters is shown in Figure 2.  The topology for linking electric meters to the telephone network is shown in Figure 3, as a subset of the general California model.  Interfaces and devices addressed within this implementation are indicated by white blocks and solid lines.  Null devices are cross-hatched.  Devices and interfaces not addressed are shaded gray.  Functionally, this implementation is a meter connected directly to the telephone Wide Area Network (WAN).  Consequently, the meter mount at customer premises and the LAN are in gray.

Reference Model

The system described is based upon the standard reference model, also set forth in IEEE Std. 1397.  The reference model described in IEEE 1397 is shown in Figure 4, along with some editorial lines of demarcation.  The specific telephone implementation addresses the physical layer interface up through the analysis layer, the layers enclosed within the heavy dotted lines in Figure 4.  The remaining layers, enclosed within the thin double dotted lines, from storage through the primary element, are typically enclosed within the meter, so they are left to the manufacturer’s discretion.

Recommended Standards for Interface 3

Specific assignment of standards for the services (layers) being provided shall be as follows-

Physical and Data Link layers:	IEEE 1390, Standard For Automatic Meter Reading via Telephone

IEEE P1390.1, Standard For Automatic Meter Reading via Telephone – Profile Protocols for Switched Telephone Network *

IEEE 1390.2, Standard For Automatic Meter Reading via Telephone – Network to Telemetry Interface Unit

Network layer:	RFC 791 Internet Protocol, IP v4

Transport layer:	RFC 793 Transmission Control Protocol

Application Layer:	IS0 9506-1,2 Manufacturing Message Specification-Part1: Service

Definition; Part 2: Protocol Specification

Analysis Service:	ANSI C12.19 = IEEE1377 = Utility Industry End Device Data

Tables = ”Tucker Tables”

Note: An asterisk (*) indicates that this standards is under development.

Figure 1 – IEEE1397 Topology

Figure 2 – IEEE Topology Adapted for California Electric Meters

Figure 3 – IEEE1397 Implementation Connecting Electric Meters to Telephone

Figure 4 – IEEE 1397 End Device Reference Model