Data Dictionary for CMEP and EDI Transaction Set 867

Draft:  Jim Price, ORA, 8/21/98

The following table documents the data content and translation of CMEP data elements for meter usage data to EDI Transaction Set 867.  The first section lists data elements that are in active use and would be implemented in EDI.  CMEP code values that are not in active use, or that appear in UIG’s EDI implementation guideline but do not have an equivalent in CMEP are in a second section, in case needs are identified during further discussion.  Additions to UIG’s guideline are in italics.

| Data Element | CMEP Field | Values | EDI Data Element | Values | ANSI Definition of Values | Explanation of Values |
| --- | --- | --- | --- | --- | --- | --- |
| Transaction Number |  |  | Reference Identification (BPT02) |  |  | A unique transaction identification number assigned by the originator |
| Original Transaction Number > Applies only to corrected data |  |  | Reference Identification (BPT09) |  |  | Transaction number from BPT02 of a transaction that is being corrected |
| Data Format Version | Record Version |  | Version/ Release (GS08) | 003070 |  |  |
| Type of Data | Record Type | Interval values (MEPMD01) | Report Type (BPT04) | C1 | Interval readings | Interval valuess |
|  |  | Cumulative values (MEPMD02) |  | DD | Usage | Cumulative values |
| Purpose of Transaction. Indicates the reason for this data transmission. Defined CMEP values are: * Original Transmission | Purpose | OK (Normal transmission) | Transaction Set Purpose Code (BPT01) | 00 | Original | Normal transmission |
| * Resend (Retransmission of previously sent data) |  | RESEND |  | 07 | Duplicate | Retransmission of previously furnished information |
| * History (Archival account data, retrieved from long term storage and maybe of lesser time resolution than its original collection period.) |  | HISTORY |  | 52 | Response to Historical Inquiry | History |
|  |  | RESEND |  | CO | Correction | Previously reported readings are being corrected |
| Sender. Identity of the entity sending this record. | Sender ID |  | Energy Service Provider (ESP) | N101=SJ, N104=DUNS# |  |  |
| Sender’s Account #. Sender’s identification reference for the account to which this record applies. | Sender Customer ID |  | ESP’s Account # | REF01=11, REF02=Acct# |  |  |
| Receiver. Identity of the intended recipient entity of this record. | Receiver ID |  | Utility (UDC) | N101=8S, N104=DUNS# |  |  |
| Receiver’s Account #. Receiving entity’s identification reference for account to which this record applies. | Receiver Customer ID |  | UDC’s Account # | N101=8R, N104=Acct# |  |  |
| Meter Number. Physical identifier or faceplate serial number, e.g., a meter manufacturer's serial number. This field may optionally be used as a channel identifier. | Meter ID |  | Meter Number (REF02) | REF01=MG |  | Use REF02=UNMETERED for non-metered service delivery points (streetlights, etc.) |
| Service Delivery Point |  |  | Metering Location | N101=MQ, N103=20, N104=SDP# |  |  |
| Commodity. Describes what commodity type this account is for. | Commodity | E, G, W, S | Product Number (REF02) | REF01=OZ |  | Type of commodity |
| Number of Data Items | Count. Indicates the number of Date/Time, flag, & value sets to follow. |  | Number of Line Items (CTT01), Number of Included Segments (SE01) |  |  |  |
| Data Value | Data Value |  | Quantity (QTY02) |  |  | Quantity of usage |
|  |  |  | Measurement (MEA06) |  |  | Register values |
| Units of Measurement. Describes the units of the data values. CMEP uses a single field to combine three concepts: * Units of measurement * Type of value - quantity vs. cumulative total * Direction of delivery | Units | KWHREG | Unit or Basis for Measurement Code - register values for delivered quantities (MEA04 w/ QTY01=QD) | KH | Kilowatt Hour |  |
|  |  | KVAHREG |  | K5 | Kilovolt Amperes Reactive |  |
|  |  | KVARHREG |  | K3 | Kilovolt Amperes Reactive Hour |  |
|  |  | GASREG |  | CF | Cubic Feet |  |
|  |  | PULSE | Unit or Basis for Measurement Code - delivered quantities (QTY03 w/ QTY01=QD) | 1N | Count | Pulse data |
|  |  | VOLTS |  | 70 | Volt |  |
|  |  | BTU |  | BY | British Thermal Unit (BTU) |  |
|  |  | CF |  | CF | Cubic Feet |  |
|  |  | $ |  | EA | Each |  |
|  |  | CCF |  | HH | Hundred Cubic Feet |  |
|  |  | KW |  | K1 | Kilowatt Demand |  |
|  |  | KVAR |  | K2 | Kilovolt Amperes Reactive Demand |  |
|  |  | KVARH |  | K3 | Kilovolt Amperes Reactive Hour |  |
|  |  | KVA |  | K4 | Kilovolt Amperes |  |
|  |  | KVAH |  | K5 | Kilovolt Amperes Reactive |  |
|  |  | KWH |  | KH | Kilowatt Hour |  |
|  |  | THERM |  | TD | Therms |  |
|  |  | MCF |  | TZ | Thousand Cubic Feet |  |
|  |  | GKW | Unit or Basis for Measurement Code - received quantities (QTY03 w/ QTY01=87) | K1 | Kilowatt Demand |  |
|  |  | GKVAR |  | K2 | Kilovolt Amperes Reactive Demand |  |
|  |  | GKVARH |  | K3 | Kilovolt Amperes Reactive Hour |  |
|  |  | GKVA |  | K4 | Kilovolt Amperes |  |
|  |  | GKVAH |  | K5 | Kilovolt Amperes Reactive |  |
|  |  | GKWH |  | KH | Kilowatt Hour |  |
| Calculation Constant. Defines an optional value which is used as a multiplier to convert data values to engineering units. Typically this parameter is used with "pulse" data to allow calculation of equivalent "kWh" and "therm" values. | Calculation Constant |  | Meter Constant (MEA03) | MEA02=MU |  |  |
| Temperature. Affects value of generated power, as well as gas volume. |  |  | Temperature (MEA03) | MEA02=TC |  | Degrees Fahrenheit? Celsius? |
| Meter Type/ Interval. Describes the time interval between readings. Metering data is transmitted as Date/Time and value pairs. If the time interval for the data values is regular, Date/Time information past the first reading is redundant. If a Date/Time field after the first reading is empty, it is calculated by adding this interval to the Date/Time of the previous value. This field is optional if Date/Time fields are supplied for all values. > Applies only to interval data | Interval |  | Meter Type (REF02) | REF01=MT |  | Examples: KH060 = hourly interval kWh, KHTOU = TOU |
| Time Stamp. Date and Time that this record was created. | Time Stamp |  | Date & Time (BPT03 & BPT08) |  |  |  |
| Data Start Time. Describes date and time that begins the data interval. > Applies only to non-interval data | Data Start Time |  | Date Time Period (DTM07) | DTM01 = 150 | Service Period Start |  |
| Data End Time. Describes date and time that ends the data interval. > Applies only to non-interval data | Data Timestamp |  | Date Time Period (DTM07) | DTM01 = 151 | Service Period End |  |
| Data Date/ Time. When an "Interval" field is supplied, Date/Time fields after the first may left empty to be calculated when the record is read, by adding the time interval described in the "Interval" field to the supplied or calculated Date/Time value of the previous entry pair in this record. > Applies only to interval data | Data Date/ Time |  | Date Time Period (DTM07) | DTM01 = 151 | Service Period End | Ending time of interval |
| Purpose of Reading. Indicates when a meter read is an opening or closing read, and states the associated reason. > Required only for opening and closing reads. | Open/ Closing Flag | C (change meter, remove read) | Measurement Reference Code (DTM01 w/ MEA01 = BC, “Billed Actual”) | 514 | Exchanged meter read date | Change meter, remove read |
|  |  | X (remove meter) |  | 319 | Meter failure date | Meter has been removed and not replaced |
|  |  | F (final read, close of account) |  | 146 | Closing Date | Final read, close of account |
|  |  | S (credit turn off read) |  | 329 | Terminated | Credit turn off read |
|  |  | P (change meter, set read) | Measurement Reference Code (DTM01 w/ MEA01 = R1, “Opening Reading”) | 152 | Effective Date of Change | Change meter, set read |
|  |  | E (new service, set read) |  | 198 | Completion | New service, set read |
|  |  | T (turn on initial read) |  | 145 | Opening Date | Turn on initial read |
|  |  | Z (reset read, manual set to a value) |  | 114 | Tooling Required | Reset read, manual set to a value |
| Season. This identifies the season for which the values apply. | Season Identifier | S | Measurement Significance Code (MEA07) | 57 | Summer |  |
| > Applies only to non-interval data |  | W |  | 58 | Winter |  |
| Time-Of-Use component label > Applies only to non-interval data | Data Label | ON-PEAK | Measurement Significance Code (MEA07) | 42 | On Peak |  |
|  |  | OFF-PEAK |  | 41 | Off Peak |  |
|  |  | PART-PEAK |  | 43 | Mid Peak |  |
|  |  | PEAK-2 |  | 63 | Peak-2 |  |
|  |  | PEAK-3 |  | 64 | Peak-3 |  |
|  |  | PEAK-4 |  | 65 | Peak-4 |  |
|  |  | TOTAL |  | 51 | Total |  |
|  |  |  |  | 44 | Average |  |
|  |  |  |  | 45 | Summer On Peak |  |
|  |  |  |  | 49 | Winter On Peak |  |
|  |  |  |  | 50 | Winter Mid Peak |  |
|  |  |  |  | 52 | Winter Super Off Peak |  |
|  |  |  |  | 71 | Summer Super On Peak |  |
|  |  |  |  | 72 | Summer Super Off Peak |  |
|  |  |  |  | 73 | Summer Off Peak |  |
|  |  |  |  | 74 | Summer Mid Peak |  |
|  |  |  |  | 75 | Winter Off Peak |  |
| Data Quality. Data quality flags indicate the raw, estimated, valid, etc. status of values transmitted. | Data Quality | (empty) (OK) | Measurement Significance Code (MEA07) | (empty) | (OK) | Normal transmission |
|  |  | E (estimated) |  | 46 | Estimated | Estimated |
|  |  |  |  | 03 | Approximately | Estimated value due to factors beyond control of MDMA - not included in performance standard |
|  |  | A (adjusted, to correct metering inconsistency or errors.) |  | 88 | Adjusted | Adjusted to correct metering inconsistencies or errors |
|  |  |  |  | 22 | Actual | Replacement of previously estimated data |
|  |  | N (empty - may be the first entry for a new account.) |  | 10 | Not equal to | Empty - may be sent as the first entry for a new account |
|  |  | R (raw - no validation has been performed on value.) |  | 68 | As Is | Raw - no validation has been performed on value |
|  |  | V (verified) |  | 40 | Uncorrected | Verified - data is actual but appears anomalous |

Codes Not in Active Use

| Data Element | CMEP Field | Values | EDI Data Element | Values |  |  | ANSI Definition of Values |  |  | Explanation of Values |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose of Transaction. Defined values are: | Purpose |  | Purpose Code (BPT01) | 01 |  |  | Cancellation |  |  |  |  |  |
|  |  |  |  | 05 |  |  | Replace |  |  | Previously cancelled readings are being replaced |  |  |
| * Summary (Summary of scheduling point totaled data, such as monthly totals calculated from 15 minute or daily meter readings.) |  | SUMMARY | Report Type Code (BPT04) |  |  |  |  |  |  |  |  |  |
| * Profile (Account usage profile data.) |  | PROFILE |  | 22 |  |  | Functional plan |  |  | Usage model information for an aggregated customer class |  |  |
| * Template (Account usage template data.) |  | TEMPLATE |  | 23 |  |  | Contractual plan |  |  | Load template for an individual customer’s usage within an aggregated customer class |  |  |
| Units of Measurement. |  | GAL | Measurement Code (MEA04, QTY03) |  | GA |  |  | Gallon |  |  |  |  |
|  |  | ACFT |  |  | ZZ |  |  | Mutually Defined |  |  |  |  |
|  |  | LBS |  |  | LB |  |  | Pound |  |  |  |  |
|  |  |  |  |  | 2I |  |  | BTUs Per Hour |  |  |  |  |
|  |  |  |  |  | 99 |  |  | Watt |  |  |  |  |
|  |  |  |  |  | BZ |  |  | Million BTUs |  |  |  |  |
|  |  |  |  |  | DA |  |  | Days |  |  |  |  |
|  |  |  |  |  | HJ |  |  | Horsepower |  |  |  |  |
|  |  |  |  |  | K7 |  |  | Kilowatt |  |  |  |  |
|  |  |  |  |  | MO |  |  | Months |  |  |  |  |
|  |  |  |  |  | T1 |  |  | Thousand Pounds Gross |  |  |  |  |
|  |  |  |  |  | T9 |  |  | Thousand kWh |  |  |  |  |
|  |  |  |  |  | TH |  |  | Thousand |  |  |  |  |
|  |  |  |  |  | UN |  |  | Unit |  |  |  |  |
|  |  |  |  |  | WK |  |  | Week |  |  |  |  |
|  |  |  |  |  | YR |  |  | Years |  |  |  |  |
| Data End Time. |  |  | DTM06 |  |  | DTM01 = 634 |  |  | Next meter read date |  |  |  |
|  |  |  |  |  |  | DTM01 = 730 |  |  | Reporting cycle date or period |  |  |  |
| Meter Type |  |  | Unmetered Service |  | REF01=SI |  |  |  |  |  |  |  |
| Data Quality. Data quality flags are used to indicate the raw, estimated, valid, etc. status of values transmitted. |  |  | Measurement Significance Code (MEA07) |  | 31 |  |  | Calculated |  |  |  |  |
|  |  |  |  |  | 34 |  |  | Ratchet |  |  |  |  |
|  |  |  |  |  | 39 |  |  | Corrected |  |  |  |  |
|  |  |  |  |  | 62 |  |  | Current |  |  |  |  |
|  |  |  |  |  | 93 |  |  | Previous |  |  |  |  |
|  |  |  | Measurement Reference ID (MEA01) |  | AA |  |  | Beginning Actual/ Ending Actual |  |  |  |  |
|  |  |  |  |  | AB |  |  | Average Balance (average or contract demand) |  |  |  |  |
|  |  |  |  |  | AE |  |  | Beginning Actual/ Ending Estimated |  |  |  |  |
|  |  |  |  |  | AF |  |  | Actual Total |  |  |  |  |
|  |  |  |  |  | BN |  |  | Billed Minimum |  |  |  |  |
|  |  |  |  |  | BO |  |  | Meter Reading as Billed (based on contractual agreement, not actual usage) |  |  |  |  |
|  |  |  |  |  | CF |  |  | Conversion Factor |  |  |  |  |
|  |  |  |  |  | DT |  |  | Dimensional Tolerance |  |  |  |  |
|  |  |  |  |  | EA |  |  | Beginning Estimated/ Ending Actual |  |  |  |  |
|  |  |  |  |  | EE |  |  | Beginning Estimated/ Ending Estimated |  |  |  |  |
|  |  |  |  |  | TI |  |  | Time |  |  |  |  |

Kilovolt-Ampere-Reactive demand is usually expressed as peak value in time interval.  Values may be positive or negative depending upon power factor.  Values are positive for VARs produced by customer or negative for VARs consumed by customer.  Induction motors consume watts and VARs.  A condenser bank produces VARs.  An over-excited generator produces watts and VARs.  An under-excited generator produces watts and consumes VARs.

If acre-feet is needed as a permanent code, a different value should be defined and proposed to UIG.