Date Submitted:  01/02/99

Business Requirements Overview

EDI Implementation Project

EDI Implementation Summary

| Overall Recommendation and Summary |  |
| --- | --- |

Background:  Communications from Meter Data Management Agents (MDMAs) occur to enable customer energy use and service billing.  Most information transmitted by MDMAs consists of meter reading and energy use data about specific customers.  These records are typically generated by the MDMA and supplied to the Utility Distribution Company (UDC) and Energy Service Provider (ESP);  exceptions are that (1) in the case of shut-off or turn-on, the UDC may be responsible for generating appropriate records with final and initial meter readings, and (2) in the case of a meter change, the party changing out the meter may be responsible for providing the final and initial meter readings to the appropriate MDMA.

The “Permanent Standards Working Group” (PSWG), which was formed pursuant to California Public Utilities Commission (CPUC) Decision 97-12-048, has recommended migration of the data format used for communications by MDMAs from the current California Meter Exchange Protocol (CMEP) to the Electronic Data Interchange (EDI) format, particularly using implementation guidelines developed by the Utility Industry Group (UIG).  As part of the overall responsibilities of a MDMA, four tasks relate to the content and flow of EDI communications from an MDMA to other market participants:

1.	Provide the UDC and ESP (or their designated agents) access to valid meter data on the MDMA server on a scheduled basis, in CPUC-approved format.  ESPs may provide access to their respective Scheduling Coordinators via access to their mailbox.

2.	Resend previously posted data as required (e.g., adjusted, reframed, or otherwise corrected), including (a) flags for altered data points and data sets and (b) reason codes for additional explanation of adjusted, reframed, or otherwise corrected data.

3.	Provide special meter reads, upon request.

4.	Provide the most recent 12 months of customer consumption data upon request to the customer or, if authorized by the customer, to any ESP or the UDC.  The purpose of this historical data is to provide a basis for validation, estimation, and editing (VEE) of meter usage data by an MDMA as well as for tasks such as load forecasting provided by an ESP.  Transferring a customer’s usage history to its new ESP is the responsibility of the customer’s previous ESP when a customer switches ESPs.

Data provided by all four of these tasks include customer-associated identifiers, flags for estimated data, and date and time of reads and of posting.

Capability Description

Functions:

The overall MDMA business functions include Meter Reading (MR) and Meter Data Management (MDM) functions.  The PSWG’s July 29, 1998, report to the CPUC summarizes the business functions of the MDMA as follows:

Functions performed by a Meter Data Management entity:

A.	Accept raw meter reads from meter reading entity

B.	As necessary, translate data into format for internal processing

C.	Associate meter reads with customer identifiers for use in validation or estimation, if needed

D.	Validate, edit, and estimate (VEE) data

E.	If necessary, translate data into CPUC-approved format prior to posting to MDMA Server

F.	Post validated, edited, and estimated data to MDMA Server for retrieval by market participants

- Perform data adjustments as required

- Re-frame data as required

- Resend previously posted data as required

- Perform other activities as described in Section VI.7 and Appendix C-VEE of the 7/29/98 Permanent Standards Working Group report

G.	Maintain MDMA Server, as required by D.97-12-048 (e.g., guidelines for server performance and disaster recovery)

H.	Archive raw data and validated data for 36 months

Functions performed by a Meter Reading entity:

A.	Collect data at the meter, including routine meter reads, special reads, and date and time of reads

B.	Transport data to the MDM

C.	Perform any Validation that is required to be performed either on-site or at the time of reading

D.	Check for and report suspected energy theft

E.	Check for and report hazardous conditions (if meter reading is performed locally)

Additional PSWG recommendations address the performance of portions of a MDMA’s functions by entities other than the MDMA, requirements for technical/ business support to ESPs and UDCs, and performance standards relating to the quality and timeliness of posted data.

These tasks can be summarized as including the four tasks listed above under “Overall Recommendation and Summary”, plus additional tasks involving an MDMA’s participation in the processes of meter installation, testing, and maintenance, which would occur outside the mechanisms used for transmission of meter data and therefore should be addressed separately from the above-listed tasks:

- Within five days after installation of a meter, confirm that the meter and meter reading system is working properly and that the billing data gathered is valid.

- Report suspected energy theft, and report known hazardous conditions.

- Notify the ESP and UDC if a customer has irregular usage and would not be subject to the usual VEE rules.

In addition, good business practice requires the maintenance of log files.

Provide tools to measure and manage service level performance.

Operational Date

We must be fully operational by 10/1/99 in order to meet the following interface test schedules:

- List Significant Milestones

- EXAMPLE: 10/1/99-12/1/99  MDMA acceptance test for interfaces with 3rd party MDMAs

Reason for Capability

- List relevant decisions

Example: AB 1890 and CPUC Unbundling Decision of May 6, 1997.

Critical Assumptions/Risk Assessment

- Quality indicators for validated, estimated, and edited read information will be required.

- Historical usage measurement information will be stored on the MDMA server for external clients for 5 days then archived for 3 years.

- Historical Usage Measurement information for internal clients will be stored for XXX year online then archived for XXX years.

- All meter reads will include a date and time stamp.

Usage deliver to the MDMA server assumes that only complete, validated usage and account information will be provided.

Sensitivity Analysis

This capability has been mandated by the CPUC.  Failure to provide the baseline functionality will prohibit market participants from exchanging usage data in the standard manner adopted by market participants and approved by the CPUC. ????????????????????????

Alternatives Considered

Risk of Postponing/Eliminating

Failure to provide a mechanism of exchanging usage information with market participants will limits the ability for all concerned to use a standard format.  The risk of being unable to support clients as required by the CPUC includes:

Failure to provide this capability will cause a cascade of major impacts and potentially numerous unnecessary alternative efforts by all participants in the market.

Method for Measuring Success

Success can be measured by integrated testing to answer the following questions:

- Can we deliver measured usage to the MDMA server in the MDMA format to the appropriate mailbox?

- Can we deliver measured usage in a consistent way to all clients?

- Can we archive and retrieve data within the appropriate time frames?

- Can we grant access to external parties to the MDMA server and handle any technical problems with this connection?

- Can we track effective dates for MDMA account and meter assignments?

- Can we assess our service level performance?

- Have we met the MDMA performance criteria?

Key Participants

List participants who have contributed to the development of these guidelines with intentions of participating in the market

Contingencies

Adhoc delivery of usage data could be achieved using techniques and formats used today to exchange information between internal systems.

Identify alternatives for exception processing:

Functional Breakdown of Business Requirement Packages

The conduct of these activities is subject to Rule 22 (D.97-10-087, Appendix A, subsequently modified by D.97-12-048), particularly Section H(7).  The performance of MDMA services is to be performed in accordance with CPUC regulations and will be the responsibility of the party so indicated in the customer's DASR.  MDMA obligations include but are not limited to the following:

(a)	Meter data for DA Customers shall be read, validated, edited, and transferred pursuant to Commission-approved standards.

(b)	Regardless of whether ESP or UDC perform MDMA services both UDC and ESP shall have access to the MDMA server.

(c)	The MDMA shall provide reasonable and timely access to Meter Data as required to allow the proper performance of billing, settlement, scheduling, forecasting and other functions.

(d)	The MDMA is required to keep the most recent 12 months of Customer consumption data for each DA Customer.  Such data must be retained for a period of 36 months.  Such data must be released on request to the customer or, if authorized by the customer, to any ESP or to the UDC.  An ESP initiates this by contacting the customer’s current ESP.

(e)	Within five days after installation of the meter, the MDMA must confirm that the meter and meter reading system is working properly and that the billing data gathered is valid.  Mechanisms for this process, and for verification that meter numbers match in customer account vs. meter usage databases, need further definition.

(f)	Either no more than 10% of service accounts read will contain estimated data, or no more than 1% of all data will be estimated.

Additional tariff requirements for related activities are stated in sections M (Meter Reading Data Obligation), H (Metering Services), and I(6) of Rule 22.

Identify Listing of flow charts, function requirements, formats (867), VEE rules, etc

Date Submitted:

Business Requirements Package

EDI Implementation Project

EDI Requirement

1. Scope

The requirements defined below and in the attached supporting documentation describe the business requirements relative to business requirements for the EDI Implementation.   These requirements outline a delivery system to be used to pass settlement quality usage data collected by market participants and clients.

2. Referenced Documents

- Attachment A:	Process Flow Diagram

- Attachment B:	MDMA Server Technical Requirements

- Attachment C:	MDMA File Specification

- Attachment D:	MDMA Security Recommendation

- Attachment E:	Monthly Read Validation, Editing, and Estimation Requirements

- Attachment F:	Interval Read Validation, Editing, and Estimation Requirements

- Attachment G:	MDMA Acceptance and Performance Criteria

3. Requirements

3.1 Mandatory 12/1/99 Requirement: Yes

3.2 Date Required:

We must be fully operational by 12/1/99 in order to meet the following interface test schedules:

- 10/1/98-11/1/99   MDMA acceptance test for interfaces with all MDMAs

3.3 General Statement of Requirements

Meter Data Management Agents (MDMAs) have the requirement to collect, validate, estimate if necessary, and deliver usage (identify source) addresses the data delivery requirements and assumes that validating, estimating and editing of the data has already occurred.  Much of the groundwork for these requirements was laid down through participation in the MDCS workshops and MADAWG technical teams.

(Identify resource) requires that transfer of usage information to clients occur through a common, central repository in a common format.  MDCS workshop requires strict performance guidelines which will need to be measured and reported to outside parties (is DQI addressing this point?????).

3.4 Interface Requirements

- Internet addressable MDMA server

- Technical support

3.5 Inter-System Interface/Dependency Requirements

(For market participants internal use only)

3.6 Security/Privacy Requirements

- Need to address

3.7 Specific Implementation Requirements

Functions:

- Provide meter reads and/or usage data along with basic account and meter information to clients. Clients may be 3rd parties such as ESPs that are providing some Direct Access service(s) to the customer.

- Technical Support Desk (use terminology from decision)

- Support archiving of data (list requirements)

- Provide technical system interface specifications and end user documentation.

- Provide tools to measure and manage service level performance.List and define…include DQI recommendations and monthly reporting requirements.  (Note:  This will expedite reporting requirements if we can automate)

1.  Calculated usage.

2.  Reads

3.  Account identification such as site, installed service, meter, customer,

Ad Hoc

1.  Client MDMA account data

2.  Request for technical support

Archiving

- Archive Retrieval request

- Reads/Usage measurement

- Exceptions

- Archive retrieval

Routine

- Delivery Schedule

- MDMA Quantity of estimated usage

- MDMA log file

- Who retrieved

- How long/when/what

- Archive retrievals

Exceptions

1.  Exceptions:

2.  Reads and Usage Measurement

Data Management

- Service modifications to MDMA client account

- Security

- Meter changes

- ESP changes

- Meter reading agent changes

Reply to other parties satisfying their request

Archive retrieval request

Log File entries

- MDMA Interface file

- Archive data

- Log File entries

Performance

- Quantity of estimated metrics

- Did we meet the schedule

- Who/what/when/how long data retrieved

- Server availability

4. Related Capabilities

Interfaces with 650, 814, etc

5. Assumptions

- Relationship between ESP, MDMA, MSP, and UDC (identify priority and working communications/relationships

- Service Agreements

- Quality indicators for validated, estimated, and edited read information will be required.

- Need to address anticipated volume

- All meter reads will include a date and time stamp.

- Usage deliver to the MDMA server assumes that only complete, validated usage and account information will be provided.

- Street lights (unmetered)

6. Glossary

7. Notes

8. Appendices

Issues with technical group resolution

[TO BE INSERTED, FROM MEETING NOTES]

Generic Descriptions

1	ISSUE:

RESOLUTION:

ALTERNATIVES:

REASON:

Technical Data Issues

1	ISSUE:	Example:  Data Format

RESOLUTION:

ALTERNATIVES:

REASON:

Attachment A: Process Flow Diagram

[DIAGRAMS TO BE INSERTED]

As described in “Overall Recommendations and Summary”, four tasks relate to the content and flow of communications from an MDMA to other market participants and constitute the scope of this project:

1.	Provide the UDC and ESP (or their designated agents) access to valid meter data on the MDMA server on a scheduled basis, in CPUC-approved format.  ESPs may provide access to their respective Scheduling Coordinators via access to their mailbox.

2.	Resend previously posted data as required (e.g., adjusted, reframed, or otherwise corrected), including (a) flags for altered data points and data sets and (b) reason codes for additional explanation of adjusted, reframed, or otherwise corrected data.

3.	Provide special meter reads, upon request.

4.	Provide the most recent 12 months of customer consumption data upon request to the customer or, if authorized by the customer, to any ESP or the UDC.  The purpose of this historical data is to provide a basis for validation, estimation, and editing (VEE) of meter usage data by an MDMA as well as for tasks such as load forecasting provided by an ESP.  Transferring a customer’s usage history to its new ESP is the responsibility of the customer’s previous ESP when a customer switches ESPs.

These tasks can generally be categorized as “routine” activities or as processing of “exceptions” (including processing that occurs “upon request”), with flows of information depicted in the following diagrams.

Task 1 (as listed above) is a “routine” activity, as shown.  Starting when a customer signs up with an ESP, the MDMA sends meter reads to the ESP and UDC until the customer leaves the ESP or the ESP assigns a different MDMA.  This is preceded by a notification of the customer sign-up from the ESP.  (This is parallel to other responses that are triggered by a customer sign-up:  sending customer service-related account information and billing information between the ESP and UDC, and sending meter-related information to and from the MSP.)

Task 2 deals with “exceptions”, as shown except that the MDMA may discover a problem itself (in which case the notification of missing or inaccurate usage would occur internally) in addition to events where the ESP, MSP, or UDC notifies the MDMA of needs for resending data due to reframing, adjustment, or other correction.  General agreement in CA is that the MDMA should always post in complete “billing periods”.  However, resending data may be needed by filename, transaction number, date of posting, or time period.  Reasons for resending data can include:

- Data was posted with gaps between billing cycles – MDMA goes back to correct and fill in gaps

- MDMA was unable to read meter and posts estimated data, but later gets and posts actual data from meter.

- MDMA posts adjusted data.  This is typically triggered by a meter investigation due to validation errors (typically initiated by MDMA?), a routine meter check or test initiated by the MSP, a customer complaint, or a meter investigation due to ESP or UDC request.  The following adjustment situations can occur:

- Crossed Meters – Usage for one customer is being registered and billed on another customer's meter.

- Cut-In-Flat - Adjustment for a customer who has received unmetered service (free usage).

- DR Meter - Adjustment for a meter that doesn't register.

- Fast Meter - Adjustment for a meter that is registering fast.

- Slow Meter - Adjustment for a meter that is registering slow.

- Dropped phases -This could also include a meter that has dropped one or more phases.

- Fixed Usage - Adjustment for an account due to incorrect establishment of fixed usage load.

- Unmetered - Adjustment for an account billed with incorrect usage

- Over-Read/Under-Read - (meter reader error, etc.)

- Incorrect multipliers applied

- ESP or UDC does not think they received meter usage data they need to bill a specific customer or meter and asks the MDMA to resend data for that specific customer or meter

- ESP or UDC either didn’t pick up file or “lost” it and needs the data resent.

- Data fails to pass a recipient’s validation, and the recipient requests the MDMA to re-VEE the data.  The MDMA rechecks the data.  If the MDMA finds there was an error, the MDMA sends the revised data for the requested period.  If there are no changes, the MDMA should add “verified” flags to any suspicious data points, or notify the requester by phone or other means.

Tasks 3 and 4 are “exceptions” that occur “upon request”.  Task 3 requires a new (“special”) meter read (e.g., pick-up reads), for reasons including the following:

- “Read” (or billing) cycle changes for customer

- Customer changes tariff

- MSP activities (e.g., replacing failed meters) and other meter/account changes, starts, stops, etc.  (Note:  in the following, “read” includes whatever data is required for billing for that meter – may include interval data, TOU, demand, etc, not just the traditional meter reading)

- Meter changeout – remove read, set read

- Meter installed, new service – set read

- Meter removed, not replaced

- Account close – final read

- Credit turn off/turn on reads

- New account – initial read

- Reset read – manually set to a value

- IDR meter has “hazardous” condition.  In some cases, the UDC replaces it with a monthly meter.  The ESP then installs a new IDR meter.

- Off-cycle read request  (all data types – interval, demand, TOU, usage)

Task 4 involves a data source that is internal to the MDMA instead of being retrieved from a meter, in these instances:

- When a customer signs up or ends service with an ESP, send usage history (12 months) and opening and closing reads to affected ESPs.  (automatic request)

- Send usage history to ESPs with customer authorization, for a specific time period.

All data posted to either the ESP or UDC will be provided.  This may include:

- Interval consumption, if available.  15-minute intervals should be used if available.

- Monthly cumulative consumption and demand.

- TOU billing determinants by TOU period (on-peak, mid-peak, off-peak, and super-off-peak).  Summer and winter seasons within a billing period have separate TOU periods.

- KVAR data, if available.

The provided data should be for the specific customer at the specific site;  a change of customer at a site, and a change of address for a customer, cause a new starting point for the availability of data.

Data Requirements

The values transmitted in Metering Service Data Records are typically an accumulation of some quantity, such as kilowatt-hours, over an interval of time.  There are two broad categories of metering data intervals employed for different accounts.  Which category of metering data interval applies to a particular account depends on the type of meter (or on billing requirements) rather than which of the routine or “exception” tasks (as described above) is being performed.  The first category is where accounts are billed on interval increments of energy usage such as an hourly or 15-minute basis.   The second is what is traditionally known as “cumulative” and consists of the following commodities, which may be broken into as many as five or more components:

- Metered usage

- Metered demand

- Meter readings

- Unmetered usage

- Unmetered demand

In addition to validated data in these categories, MDMAs may send the following in either interval or cumulative (usage or demand, TOU or non-TOU) formats:

- Raw pulse data

- “Pre-VEE” engineering units data

Each value has an associated Date/Time field as a timestamp.  That timestamp could conceivably identify the beginning or ending time of the associated time interval;  current practice in California uses end-of-interval timestamps.  Nominally, data transmissions of cumulative meter reads, such as monthly meter reads, should include beginning of period and end of period dates, end of period meter reads, and usage values.  This allows receiving entities to verify that no reading overlaps or gaps have occurred.  Totaling algorithms must deal with the fact that the timestamp for the last interval of the day will be for the beginning of the first interval of the next day.

The occurrence of multiple metered commodities for a single metering point (due to VEE rules, among other reasons) for multiple metered accounts with kVARh require the MDMA to know which kVARh channels map to which kWh channels, and how the meters and channels relate to each other.

Illustrations

The following scenarios illustrate the anticipated handling of situations other than routine reporting of usage data:

- Scenario #1	Start date specified, data posted, start date changed to a later date before read date

- The MDMA is contacted and told that service for a given customer and meter is starting at a given date/time.

- The MDMA obtains the start meter read.

- On the meter’s scheduled read date, the MDMA reads the meter.  The MDMA posts the interval data for the meter from the start date/time through the scheduled read date.

- The MDMA is notified that the customer actually moved in later than originally specified.

- The MDMA reposts the customer’s interval data; the data now goes from the new (later) start date to the scheduled read date.

| April 28 | MDMA notified of new service starting May 1 |
| --- | --- |
| May 1 - ORIGINAL START DATE | MDMA starts reading interval data |
| May 3 - NEW START DATE |  |
| May 5 - SCHEDULED METER READ DATE | MDMA reads and posts interval data for 5/1 - 5/4 |
| May 7 | MDMA notified service actually started May 3 |
| May 9 | MDMA reposts interval data for 5/3 - 5/4 |

- Scenario #2	Start date specified, data posted, start date changed to a date after the read date

- The MDMA is contacted and told that service for a given customer and meter is starting at a given date/time.

- The MDMA obtains the start meter read.

- On the meter’s scheduled read date, the MDMA reads the meter.  The MDMA posts the interval data for the meter from the start date/time through the scheduled read date.

- The MDMA is notified that the customer actually moved in later than originally specified - later than the read date of the data just posted.  I.e., the data just posted is completely invalid for the customer.

- The MDMA ?????

| April 28 | MDMA notified of new service starting May 1 |
| --- | --- |
| May 1 - ORIGINAL START DATE | MDMA starts reading interval data |
| May 5 - SCHEDULED METER READ DATE | MDMA reads and posts interval data for 5/1 - 5/4 |
| May 7 - NEW START DATE |  |
| May 8 | MDMA notified service actually started May7 |
| May 10 | MDMA ???? |

- Scenario #3	Start date specified, data posted, start date changed to an earlier date

- The MDMA is contacted and told that service for a given customer and meter is starting at a given date/time.

- The MDMA obtains the start meter read.

- On the meter’s scheduled read date, the MDMA reads the meter.  The MDMA posts the interval data for the meter from the start date/time through the scheduled read date.

- The MDMA is notified that the customer actually moved in earlier than originally specified.

- If the MDMA has interval data for the new period (from the new start date), the MDMA reposts the data from the new start date to the read date.  Note that this may include more than one billing period of data.  If interval data is not available for the earlier time, the MDMA estimates the data and reposts it in a similar fashion.  In this case, the estimated intervals are marked as estimated.

| April 28 | MDMA notified of new service starting May 3 |
| --- | --- |
| May 1 - NEW START DATE |  |
| May 3 - ORIGINAL START DATE | MDMA starts reading interval data |
| May 5 - SCHEDULED METER READ DATE | MDMA reads and posts interval data for 5/1 - 5/4 |
| May 8 | MDMA notified service actually started May1 |
| May 10 | MDMA reposts data from 5/1 - 5/4 |

- Scenario #4	Stop date specified, data posted, stop date changed to a later date

- The MDMA is contacted and told that service for a given customer and meter is ending at a given date/time.

- The MDMA obtains the stop meter read.

- The MDMA posts the meter’s data from its last read schedule through the stop read.

- The MDMA is notified that the customer actually moved out later than originally specified.

- If possible, the MDMA obtains interval data for the additional time period the customer occupied the premise.  If not, the MDMA estimates the data.  The MDMA reposts the data from the meter’s last scheduled read day through the new stop date/time.  Note that this may include more than one billing cycle, depending on when the MDMA is notified of the change in stop dates.

| April 28 | MDMA notified service ending May 1 |
| --- | --- |
| May 1 - ORIGINAL STOP DATE | MDMA obtains final meter read |
| May 2 | MDMA posts final data through 5/1 |
| May 5 NEW STOP DATE |  |
| May 8 | MDMA notified service actually stopped May5 |
| May 10 | MDMA estimates data through 5/5? and reposts??? |

- Scenario #5	Stop date specified, data posted, stop date changed to an earlier date after meter’s scheduled read date

- The MDMA is contacted and told that service for a given customer and meter is ending at a given date/time.

- The MDMA obtains the stop meter read.

- The MDMA posts the meter’s data from its last read schedule through the stop read.

- The MDMA is notified that the customer actually moved out earlier than originally specified.

- The MDMA reposts the meter’s data from the its last scheduled read date through the new stop date/time.

| April 28 | MDMA notified service ending May 3 |
| --- | --- |
| May 1 - NEW STOP DATE |  |
| May 3 - ORIGINAL STOP DATE | MDMA obtains final meter read |
| May 4 | MDMA posts final data through 5/3 |
| May 8 | MDMA notified service actually stopped May 1 |
| May 10 | MDMA - reposts data through 5/1??? |

- Scenario #6	Stop date specified, data posted, stop date changed to an earlier date before the previous read date

- The MDMA is contacted and told that service for a given customer and meter is ending at a given date/time.

- The MDMA obtains the stop meter read.

- The MDMA posts the meter’s data from its last read schedule through the stop read.

- The MDMA is notified that the customer actually moved out earlier than originally specified.  The new date is prior to the last scheduled read date previously posted - i.e. none of the data in the last file sent is valid for this customer.

- The MDMA reposts the meter’s data from the previous scheduled read date that is before the new end date through the new end date.  How does the MDMA server notify UDC/ESP that the earlier data is completely invalid for this customer?  Does it need to, or is it just implied from the start/end dates?

| April 28 | MDMA notified service ending May 5 |
| --- | --- |
| May 1 - SCHEDULED READ DATE | MDMA reads and posts interval data through 4/30 |
| May 5 - ORIGINAL STOP DATE | MDMA obtains final meter read |
| May 6 | MDMA posts final data through 5/5 |
| May 8 | MDMA notified service actually stopped April 29 |
| May 10 | MDMA - reposts data through 4/29??? |

Attachment B: MDMA Server Technical Requirements

Attachment C: MDMA File Specification

Insert 867 Format here

Attachment D: MDMA Security Recommendation

Attachment E: Monthly Read Validation, Editing, and Estimation

Insert VEE Rules here

Attachment F: Interval Read Validation, Editing, and Estimation

Insert VEE Rules here

Attachment G: MDMA Acceptance and Performance Criteria

Other ??

Insert relevant decisions from CPUC here

Insert relevant decisions / recommendations from sub-groups here)

The same data format may be used to transmit load profiles, but this would not be performed by the MDMA and is beyond the scope of this business model description.