# Example EDI Transaction Data

Scenario:  An ESP’s DASR has been accepted by a UDC and the ESP intends to take on a role involving the meter (e.g., MSP, MDMA).  Meter configuration information would be automatically sent to the ESP after DASR acceptance.

|  | This meter’s face plate appears to state the following: Model SE-240 Watthour Meter Cat. No. SE0092-01 S/N SE240A0003 FM2S 240V 60Hz CL200 1W Kh7.2 TA30 That is: Model number SE-240, and catalog # SE0092-01. Serial number SE240A0003. Capability rating 240 volts and 200 amps. Calibrate at 30 amps. Physical configuration is form # 2S. Billing constant = 1, Ke = 7.2 kWh per pulse. Note that this is just the base module for this meter, and that add-on modules can add a variety of capabilities. |
| --- | --- |

An illustration of an EDI description of this meter’s installed configuration (additional details assumed as needed) is then:

| ISA*00* *00* *01*006908818 *01*006912877 *990502*1546*U*00307*000000001*0*P*~\ |  |
| --- | --- |
| GS*IN*006908818*006912877*990502*1546*1*X*003070\ | Interchange, Functional Group, and Transaction Set Control Headers |
| ST*650*0001\ |  |
| BGN*00*1999050201*990502*1546*PT*1999043101*IA***19\ | Description of Existing Meter Installation in response to ESP’s DASR # 1999043101 |
| N1*8R*JOHN DOE*91*534267346732\ | Customer, identified by utility’s account #. |
| N3*123 MAIN ST*APT. 1\ | Customer’s service address |
| N4*SUBURBIA*CA*99999\ | Customer’s city, state, zip code |
| N1*8S**1*006908818\ | Utility, identified by DUNS#. |
| N1*SJ**1*006912887\ | ESP, identified by DUNS#. |
| N1*H8**1*006912887\ | MSP, identified by DUNS # |
| REF*11*123456789\ | ESP’s account # for customer. |
| HL*1**EV*0\ | Data contains information other than new installation or removal. |
| REF*LU*176091234567890\ | Service Delivery Point ID. |
| REF*MG*SE240A0003\ | Meter serial number is SE240A0003. |
| REF*MJ*SE240\ | Meter model is SE240. |
| REF*MT*KHMON\ | Meter type is monthly kWh, as currently configured. |
| REF*FQ*2S\ | Meter form is 2S. |
| REF*IX*6\ | Meter display has 6 digits. |
| DTM*230*****D8*19990104\ | Meter was installed 1/4/99. |
| DTM*105*****D8*19990106\ | Meter was last calibrated 1/6/99. |
| YNQ**Y*******OPT\ | Meter has an optical port. |
| YNQ**N*******IDR\ | Meter is assumed to have not been configured to store interval data, although this meter model offer that option and accept add-on modules for interval data recording, radio communications, etc. |
| YNQ**N*******RADIO\ | Meter does not have radio communications. |
| MEA*EL*QC*200\ | Meter class is 200 amps. |
| MEA*EL*B1*30\ | Meter is calibrated at 30 amps. |
| MEA*EL*VO*240\ | Meter voltage is 240 volts. |
| MEA*EL*VSO*240\ | Service voltage is 240 volts. |
| MEA*EL*35*3\ | Meter has 3 wires. |
| MEA*EL*36*1\ | Meter is single phase. |
| MEA*CT*T3*7.2\ | Meter Ke is 7.2 watt-hours per pulse. |
| MEA*CT*MU*1\ | Billing constant is 1. |
| NM1*MF*2*ACME POWER SYSTEMS*****1*999999999\ | Meter manufacturer is Acme Power Systems. |
| MTX*ACC*WATCH OUT FOR THE FEISTY CHIHUAHUA.\ | Access instructions, etc. Lengthy comments can be provided here. |
| SE*32*0001\ | This transaction contains 32 data segments. |
| GE*1*1\ | Functional Group Trailer |
| IEA*1*000000001\ | Interchange Control Trailer |

The structure of this transaction is as explained in the attachment, “EDI File Structure”.

Jim Price, ORA, 10/16/98

## EDI File Structure

This attachment describes the organization of data communications using the ANSI X12 standard.

An EDI transaction can be considered as simply an ASCII text stream of data segments, with each data segment consisting of one or more data elements.  The ANSI X12 standards provide a uniform syntax (i.e., a defined set of rules) describing what each data segment and data element can contain, along with a set of standard code-table values and cross-references.  Some data segments are described as control segments and contain control information rather than the business information that ultimately is the intent of the data transmission.  These control segments provide an electronic envelope in which one or more X12 messages are carried from sender to receiver.  In particular, an EDI data transmission begins with an Interchange Control Header segment that begins with the code “ISA”, which includes information identifying the sender and receiver and specifying certain details of how the data transmission is to be processed, and ends with a matching Interchange Control Trailer.  Between the interchange control segments there are one or more Functional Groups, which begin with “GS” and end with “GE” segments, and which provide additional control information.  Several sets of data, and types of data, can be combined in a single data transmission, due to this functional control structure.

The actual “transaction” is sometimes informally referred to as a “document”, because it is an electronic equivalent of a paper document such as a Purchase Order, an Invoice, or a Report of Test Results.  It begins and ends with “ST” and “SE” control segments that define the type of document.  The actual “transaction set” then contains one or more “tables”, and is divided into three areas:  the Heading Area, the Detail Area, and the Summary Area.  The first table starts with the ST control segment, and the last table ends with the SE control segment.  Within each table, some segments may be repeated, such as reference information (which starts with “REF”) that states information such as account numbers.  When segments add information (such as addresses) to other segments (such as senders’ and receivers’ names), groups of segments may be repeated -- this is referred to as a “loop”.  The entire Detail Area is often a loop.  For example, for meter data formatted using ANSI X12 transaction set 867, a “PTD” segment begins a loop that contains data for a meter or a register over a number of recording intervals, and there may be multiple meters or registers for a single account, resulting in multiple PTD loops within a transaction set that applies to the account.  There may be nested loops within loops, such as multiple recording intervals for a meter or register, and this would be represented by multiple “Quantity” (QTY) loops within a PTD loop.

A segment contains one or more Data Elements, which is the smallest unit of information defined in the standard.  Data Elements are the actual values that give meaning to the data.  Some data elements are selected from lists of predefined values, and these often describe the meaning of other data elements within the same segment;  for example, the code KH (when used in a specific location) would indicate that the number 17324 used in the same segment represents a consumption of 17,324 kWh during the billing period.  This usage may be clarified by comparison to other EDI contexts, in which (for example) the X12 reference code TE tells the receiver that the following data item should be interpreted as a phone number rather than a fax or pager number.  The value “TE” is a standard, predefined code value, while the phone number itself is a user-supplied value.  The X12 standards provide predictable positions, or data pigeonholes, where the sender can place and the receiver can find pertinent data.